Testing and Validation – SCP

December 15, 2022

- Content

1.4	Detailed report	5

1.4.1 Test Scenario: GATEUYSISCAPII-1075- Standard Flow	5

1.4.2 Test Scenario: GATEUYSISCAPII-1076 - Boarding Pass Invalid (valid barcode)	8

1.4.3 Test Scenario: GATEUYSISCAPII-1077 - Flight Validation 1: Departure flight MVD + flight date today	11

1.4.4 Test Scenario: GATEUYSISCAPII-1078 - Flight Validation 2: Departure flight MVD + flight date tomorrow	14

1.4.5 Test Scenario: GATEUYSISCAPII-1079 - Flight Validation 3: Departure flight MVD + flight date yesterday	17

1.4.6 Test Scenario: GATEUYSISCAPII-1080 - Flight Validation 4: Departure flight not MVD + flight date today	20

1.4.7 Test Scenario: GATEUYSISCAPII-1081 - Flight Validation 5: Departure flight not MVD + flight date tomorrow	23

1.4.8 Test Scenario: GATEUYSISCAPII-1082 - Flight Validation 6: Departure flight not MVD + flight date yesterday	26

1.4.9 Test Scenario: GATEUYSISCAPII-1083 - Flight Validation 7: Departure flight MVD + flight date NULL	29

1.4.10 Test Scenario: GATEUYSISCAPII-1084 - Flight Validation 8: Departure flight not MVD + flight date NULL	32

1.4.11 Test Scenario: GATEUYSISCAPII-1085 - Flight Validation 9: Departure NULL+ flight date today	35

1.4.12 Test Scenario: GATEUYSISCAPII-1086 - Flight Validation 10: Departure NULL+ flight date tomorrow	38

1.4.13 Test Scenario: GATEUYSISCAPII-1087 - Flight Validation 11: Departure NULL+ flight date yesterday	41

1.4.14 Test Scenario: GATEUYSISCAPII-1088 - Flight Validation 12: Departure NULL+ flight date NULL	44

1.4.16 Test Scenario: GATEUYSISCAPII-1090 - Gate alarm - Tailgate	47

1.4.17 Test Scenario: GATEUYSISCAPII-1091 - Gate alarm - Doors Blocked	51

1.4.18 Test Scenario: GATEUYSISCAPII-1092 - Gate alarm - Forced doors	55

1.4.19 Test Scenario: GATEUYSISCAPII-1093 - Gate alarm - No one detected	56

1.4.20 Other test cases	59

1.4.20.1 Open and close the eGate doors from the inspector.	59

1.4.20.2 Opening the eGate doors from the inspector.	60

1.4.20.3 Restarting the gateway from the inspector	61

1.4.20.4 Abandoned object alarm	62

1.4.20.5 Person standing on the other side of the door where the sensor is located	66

1.4.20.6 NULL flight destination + date of today's flight	68

1.4.20.7 Numeric flight destination + today's flight date	70

1.4.20.8 Flight destination consisting of special characters + today's flight date	72

1.4.20.9 Destination consisting of four characters+ date of today's flight	74

1.4.20.10 Destination composed of two characters + today's flight date	76

1.4.20.11 Origin composed of two characters + today's flight date	78

1.4.20.12 Origin composed of four characters + today's flight date	80

1.4.20.13 Origin formed by numbers + today's flight date	82

1.4.20.14 Origin consisting of special characters+ today's flight date	84

1.4.20.15 Flight code null+today's flight date	86

1.4.20.16 Numeric flight code+today's flight date	88

1.4.20.17 Alphabetical flight code + today's flight date	90

1.4.20.18 Flight code made up of special characters + date of today's flight	92

1.4.20.19 Scan boarding with flight code under 8 characters	94

1.4.20.20 Scan boarding with a flight code longer than 8 characters	96

1.4.20.21 Scan boarding with a flight seat more than 4 characters long	98

1.4.20.22 Scan boarding with a flight seat less than 4 characters long	100

1.4.20.23 Scan boarding with a seat with alphabetical value	102

1.4.20.24 Scan boarding with a seat with a value equal to special characters	104

1.4.20.23 Scan boarding with a seat with a value equal to only numbers	106

1.4.20.24 Scan boarding for an already boarded passenger	108

1.4.20.25 Scan boarding of a passenger with the same seat as another passenger who does not confirm boarding	110

1.4.21 Annexes	112

Annex A	112

Control sheet

| Organization | EC Solutions |  |  |
| --- | --- | --- | --- |
| Deliverable | Testing and Validation – SCP |  |  |
| Author | Testing |  |  |
| Version/Edition | 0100 | Date Version | 15/12/2022 |
| Approved by |  | Date of Approval |  |
|  |  | Total number of pages | 112 |

Change log

| Version | Cause of Change | Responsible for Change | Date of Change |
| --- | --- | --- | --- |
| 0100 | Initial version | Ailyn del Pino Acosta | 22/11/2022 |

Distribution control

First and last names

Ailyn del Pino Acosta

# Detailed report

## 1.4.1 Test Scenario: GATEUYSISCAPII-1075- Standard Flow

Sumary

| Passenger scans a Boarding Pass for the current date |  |  |  |  |  |  |  |  |  |
| --- | --- | --- | --- | --- | --- | --- | --- | --- | --- |
| Step | Description | Data | Data 2 | Expected Result | Actual result | To be defined | Priority High, Medium or low | Passed | Responsible |
| 1 | Passenger is instructed a boarding pass | Interior Screen: SC0001 |  | System shall: -Instructing passenger to scan BCBP -Display Screen SC0001 -The eGate doors are closed -The eGate semaphore shows a green arrow in the model -The eGate side illumination are green. | The side illumination of the eGate is red. |  | High | False |  |
| 2 | Passenger presents a boarding pass | Interior Screen: SC0002 | valid boarding pass | System shall: -Display screen SC0002 informing passenger the BCBP is being read -Detect the boarding pass -Barcode reader LED: off |  | -Should the barcode reader's LED light green when it detects boarding? | High | true |  |
| 3 | Passenger waits for barcode validation |  |  | System shall: -Do a request to Service with Boardingpass Data, in order to have the validation of BCBP -Service receives the request from the system -Service responds to the system with: code, idTransactionEP and message |  |  |  |  |  |
| 4 | Service responds with "code": 0 | { "code": 0 "idTransactionEP": 0, "message": "Boarding correcto" } |  | System shall: Continue to accept transaction |  |  |  |  |  |
| Accept transaction |  |  |  |  |  |  |  |  |  |
| Step | Description | Data |  | Expected Result | Actual result | To be defined | Priority High, Medium or low | Passed | Responsible |
| 1 | Passenger is instructed to cross doors | Interior Screen: SC0005 |  | System shall: -Instruct passenger to cross doors -Open doors -Display screen SC0005 | System shall: -Instruct passenger to cross doors -Open doors -Display screen SC0005 |  |  | true |  |
| 2 | Passenger cross doors |  |  | System shall: -Close doors -Return to initial state | System shall: -Close doors -Display screen SC0006 -Return to initial state |  | High | false |  |
| 3 | Message displayed by the inspector |  |  |  |  | What message should the inspector display? |  |  |  |

## 1.4.2 Test Scenario: GATEUYSISCAPII-1076 - Boarding Pass Invalid (valid barcode)

Sumary

| Passenger scans an invalid Boarding Pass |  |  |  |  |  |  |  |  |  |
| --- | --- | --- | --- | --- | --- | --- | --- | --- | --- |
| Step | Description | Data | Data 2 | Expected Result | Actual result | To be defined | Priority High, Medium or low | Passed | Responsible |
| 1 | Passenger is instructed a boarding pass | Interior Screen: SC0001 |  | System shall: -Instructing passenger to scan BCBP -Display Screen SC0001 -The eGate doors are closed -The eGate semaphore shows a green arrow in the model -The eGate side illumination are green. | The side illumination of the eGate is red. |  | High | False |  |
| 2 | Passenger presents a boarding pass | Interior Screen: SC0002 | Invalid boarding pass with origin other than MVD | System shall: -Display screen SC0002 informing passenger the BCBP is being read -Detect the boarding pass -Barcode reader LED: off |  | -Should the barcode reader's LED light green when it detects boarding? | High | true |  |
| 3 | Passenger waits for barcode validation |  |  | System shall: -Do a request to Service with Boardingpass Data, in order to have the validation of BCBP -Service receives the request from the system -Service responds to the system with: code, idTransactionEP and message |  |  |  |  |  |
| 4 | Service responds with "code": different from 0 |  |  | System shall: Continue to reject transaction |  |  |  |  |  |
| Reject transaction |  |  |  |  |  |  |  |  |  |
| Step | Description | Data |  | Expected Result | Actual result | To be defined | Priority High, Medium or low | Passed | Responsible |
| 1 | Passenger is instructed to look for assistance | Interior Screen: SC0004 |  | System shall: -Passenger should look for assistance -Display screen SC0004 -Return to Initial state |  | -What should the system do when faced with an invalid barcode? -What message should appear on the inspector in both cases? | Medium | true |  |

## 1.4.3 Test Scenario: GATEUYSISCAPII-1077 - Flight Validation 1: Departure flight MVD + flight date today

Sumary

| Passenger scans a Boarding Pass for the current date |  |  |  |  |  |  |  |  |  |
| --- | --- | --- | --- | --- | --- | --- | --- | --- | --- |
| Step | Description | Data | Data 2 | Expected Result | Actual result | To be defined | Priority High, Medium or low | Passed | Responsible |
| 1 | Passenger is instructed a boarding pass | Interior Screen: SC0001 |  | System shall: -Instructing passenger to scan BCBP -Display Screen SC0001 -The eGate doors are closed -The eGate semaphore shows a green arrow in the model -The eGate side illumination are green. | The side illumination of the eGate is red. |  | High | False |  |
| 2 | Passenger presents a boarding pass | Interior Screen: SC0002 | valid boarding pass | System shall: -Display screen SC0002 informing passenger the BCBP is being read -Detect the boarding pass -Barcode reader LED: off |  | -Should the barcode reader's LED light green when it detects boarding? | High | true |  |
| 3 | Passenger waits for barcode validation |  |  | System shall: -Do a request to Service with Boardingpass Data, in order to have the validation of BCBP -Service receives the request from the system -Service responds to the system with: code, idTransactionEP and message |  |  |  |  |  |
| 4 | Service responds with "code": 0 | { "code": 0 "idTransactionEP": 0, "message": "Boarding correcto" } |  | System shall: Continue to accept transaction |  |  |  |  |  |
| Accept transaction |  |  |  |  |  |  |  |  |  |
| Step | Description | Data |  | Expected Result | Actual result | To be defined | Priority High, Medium or low | Passed | Responsible |
| 1 | Passenger is instructed to cross doors | Interior Screen: SC0005 |  | System shall: -Instruct passenger to cross doors -Open doors -Display screen SC0005 | System shall: -Instruct passenger to cross doors -Open doors -Display screen SC0005 |  |  | true |  |
| 2 | Passenger cross doors |  |  | System shall: -Close doors -Return to initial state | System shall: -Close doors -Display screen SC0006 -Return to initial state | What message should the inspector display? | High | false |  |

## 1.4.4 Test Scenario: GATEUYSISCAPII-1078 - Flight Validation 2: Departure flight MVD + flight date tomorrow

Sumary

| Passenger scans a boarding pass for the day after |  |  |  |  |  |  |  |  |  |
| --- | --- | --- | --- | --- | --- | --- | --- | --- | --- |
| Step | Description | Data | Data 2 | Expected Result | Actual result | To be defined | Priority High, Medium or low | Passed | Responsible |
| 1 | Passenger is instructed a boarding pass | Interior Screen: SC0001 |  | System shall: -Instructing passenger to scan BCBP -Display Screen SC0001 -The eGate doors are closed -The eGate semaphore shows a green arrow in the model -The eGate side illumination are green. | The side illumination of the eGate is red. |  | High | False |  |
| 2 | Passenger presents a boarding pass | Interior Screen: SC0002 | Boarding pass with tomorrow's date (more than 8 hours difference with respect to the flight). | System shall: -Display screen SC0002 informing passenger the BCBP is being read -Detect the boarding pass -Barcode reader LED: off | The system shall: -No SC0002 screen is displayed informing the passenger that the BCBP is being read. -The boarding pass is not detected. |  | High | false | ECSolutions |
|  |  |  | Boarding pass with tomorrow's date (less than 8 hours difference with respect to the flight). |  | UNTESTED |  |  |  |  |
| 3 | Passenger waits for barcode validation |  |  | System shall: -Do a request to Service with Boardingpass Data, in order to have the validation of BCBP -Service receives the request from the system -Service responds to the system with: code, idTransactionEP and message |  |  |  |  |  |
| 4 | Service responds with "code": different from 0 |  |  | System shall: Continue to reject transaction |  |  |  |  |  |
| Reject transaction |  |  |  |  |  |  |  |  |  |
| Step | Description | Data |  | Expected Result | Actual result | To be defined | Priority High, Medium or low | Passed | Responsible |
| 1 | Passenger is instructed to look for assistance | Interior Screen: SC0004 |  | System shall: -Passenger should look for assistance -Display screen SC0004 -Return to Initial state | System shall: -Passenger should look for assistance -SC0004 screen is not displayed -Return to Initial state | What message should the inspector display? | High | false |  |

## 1.4.5 Test Scenario: GATEUYSISCAPII-1079 - Flight Validation 3: Departure flight MVD + flight date yesterday

Sumary

| Passenger scans a boarding pass for the day before |  |  |  |  |  |  |  |  |  |
| --- | --- | --- | --- | --- | --- | --- | --- | --- | --- |
| Step | Description | Data | Data 2 | Expected Result | Actual result | To be defined | Priority High, Medium or low | Passed | Responsible |
| 1 | Passenger is instructed a boarding pass | Interior Screen: SC0001 |  | System shall: -Instructing passenger to scan BCBP -Display Screen SC0001 -The eGate doors are closed -The eGate semaphore shows a green arrow in the model -The eGate side illumination are green. | The side illumination of the eGate is red. |  | High | False |  |
| 2 | Passenger presents a boarding pass | Interior Screen: SC0002 | Boarding pass dated yesterday | System shall: -Display screen SC0002 informing passenger the BCBP is being read -Detect the boarding pass -Barcode reader LED: off | System shall: -Display screen SC0002 informing passenger the BCBP is being read -Detect the boarding pass -Barcode reader LED: off |  |  | true |  |
| 3 | Passenger waits for barcode validation |  |  | System shall: -Do a request to Service with Boardingpass Data, in order to have the validation of BCBP -Service receives the request from the system -Service responds to the system with: code, idTransactionEP and message |  |  |  |  |  |
| 4 | Service responds with "code": different from 0 |  |  | System shall: Continue to reject transaction |  |  |  |  |  |
| Reject transaction |  |  |  |  |  |  |  |  |  |
| Step | Description | Data |  | Expected Result | Actual result | To be defined | Priority High, Medium or low | Passed | Responsible |
| 1 | Passenger is instructed to look for assistance | Interior Screen: SC0004 |  | System shall: -Passenger should look for assistance -Display screen SC0004 -Return to Initial state | System shall: -Passenger should look for assistance -Display screen SC0004 -Return to Initial state | -What message should the inspector display? -What message should the inspector display when a flight is rescheduled? | High | true |  |

## 1.4.6 Test Scenario: GATEUYSISCAPII-1080 - Flight Validation 4: Departure flight not MVD + flight date today

Sumary

| Passenger scans a boarding pass for a non-MVD departure day today's date |  |  |  |  |  |  |  |  |  |
| --- | --- | --- | --- | --- | --- | --- | --- | --- | --- |
| Step | Description | Data | Data 2 | Expected Result | Actual result | To be defined | Priority High, Medium or low | Passed | Responsible |
| 1 | Passenger is instructed a boarding pass | Interior Screen: SC0001 |  | System shall: -Instructing passenger to scan BCBP -Display Screen SC0001 -The eGate doors are closed -The eGate semaphore shows a green arrow in the model -The eGate side illumination are green. | The side illumination of the eGate is red. |  | High | False |  |
| 2 | Passenger presents a boarding pass | Interior Screen: SC0002 | Boarding pass for a departure other than MVD and today's date | System shall: -Display screen SC0002 informing passenger the BCBP is being read -Detect the boarding pass -Barcode reader LED: off | System shall: -Display screen SC0002 informing passenger the BCBP is being read -Detect the boarding pass -Barcode reader LED: off |  |  | true |  |
| 3 | Passenger waits for barcode validation |  |  | System shall: -Do a request to Service with Boardingpass Data, in order to have the validation of BCBP -Service receives the request from the system -Service responds to the system with: code, idTransactionEP and message |  |  |  |  |  |
| 4 | Service responds with "code": different from 0 |  |  | System shall: Continue to reject transaction |  |  |  |  |  |
| Reject transaction |  |  |  |  |  |  |  |  |  |
| Step | Description | Data |  | Expected Result | Actual result | To be defined | Priority High, Medium or low | Passed | Responsible |
| 1 | Passenger is instructed to look for assistance | Interior Screen: SC0004 |  | System shall: -Passenger should look for assistance -Display screen SC0004 -Return to Initial state | System shall: -Instruct passenger to cross doors -Open doors -Display screen SC0005 |  | High | false |  |
| 2 |  |  |  |  | System shall: -Close doors -Display screen SC0006 -Return to initial state | What message should the inspector display? |  |  |  |

## 1.4.7 Test Scenario: GATEUYSISCAPII-1081 - Flight Validation 5: Departure flight not MVD + flight date tomorrow

Sumary

| Passenger scans a boarding pass for a non MVD departure day after departure date |  |  |  |  |  |  |  |  |  |
| --- | --- | --- | --- | --- | --- | --- | --- | --- | --- |
| Step | Description | Data | Data 2 | Expected Result | Actual result | To be defined | Priority High, Medium or low | Passed | Responsible |
| 1 | Passenger is instructed a boarding pass | Interior Screen: SC0001 |  | System shall: -Instructing passenger to scan BCBP -Display Screen SC0001 -The eGate doors are closed -The eGate semaphore shows a green arrow in the model -The eGate side illumination are green. | The side illumination of the eGate is red. |  | High | False |  |
| 2 | Passenger presents a boarding pass | Interior Screen: SC0002 | Boarding pass for a non-MVD departure one day after departure date | System shall: -Display screen SC0002 informing passenger the BCBP is being read -Detect the boarding pass -Barcode reader LED: off | The system shall: -No SC0002 screen is displayed informing the passenger that the BCBP is being read. -The boarding pass is not detected. |  | High | false | ECSolutions |
| 3 | Passenger waits for barcode validation |  |  | System shall: -Do a request to Service with Boardingpass Data, in order to have the validation of BCBP -Service receives the request from the system -Service responds to the system with: code, idTransactionEP and message |  |  |  |  |  |
| 4 | Service responds with "code": different from 0 |  |  | System shall: Continue to reject transaction |  |  |  |  |  |
| Reject transaction |  |  |  |  |  |  |  |  |  |
| Step | Description | Data |  | Expected Result | Actual result | To be defined | Priority High, Medium or low | Passed | Responsible |
| 1 | Passenger is instructed to look for assistance | Interior Screen: SC0004 |  | System shall: -Passenger should look for assistance -Display screen SC0004 -Return to Initial state | System shall: -Passenger should look for assistance -SC0004 screen is not displayed -Return to Initial state | -What message should the inspector display? -Does the departure not validate here? | High | false |  |

## 1.4.8 Test Scenario: GATEUYSISCAPII-1082 - Flight Validation 6: Departure flight not MVD + flight date yesterday

Sumary

| Passenger scans a boarding pass for a non MVD departure before departure date |  |  |  |  |  |  |  |  |  |
| --- | --- | --- | --- | --- | --- | --- | --- | --- | --- |
| Step | Description | Data | Data 2 | Expected Result | Actual result | To be defined | Priority High, Medium or low | Passed | Responsible |
| 1 | Passenger is instructed a boarding pass | Interior Screen: SC0001 |  | System shall: -Instructing passenger to scan BCBP -Display Screen SC0001 -The eGate doors are closed -The eGate semaphore shows a green arrow in the model -The eGate side illumination are green. | The side illumination of the eGate is red. |  | High | False |  |
| 2 | Passenger presents a boarding pass | Interior Screen: SC0002 | Boarding pass for a non-MVD departure one day prior to departure date | System shall: -Display screen SC0002 informing passenger the BCBP is being read -Detect the boarding pass -Barcode reader LED: off | System shall: -Display screen SC0002 informing passenger the BCBP is being read -Detect the boarding pass -Barcode reader LED: off |  |  | true |  |
| 3 | Passenger waits for barcode validation |  |  | System shall: -Do a request to Service with Boardingpass Data, in order to have the validation of BCBP -Service receives the request from the system -Service responds to the system with: code, idTransactionEP and message |  |  |  |  |  |
| 4 | Service responds with "code": different from 0 |  |  | System shall: Continue to reject transaction |  |  |  |  |  |
| Reject transaction |  |  |  |  |  |  |  |  |  |
| Step | Description | Data |  | Expected Result | Actual result | To be defined | Priority High, Medium or low | Passed | Responsible |
| 1 | Passenger is instructed to look for assistance | Interior Screen: SC0004 |  | System shall: -Passenger should look for assistance -Display screen SC0004 -Return to Initial state | System shall: -Passenger should look for assistance -Display screen SC0004 -Return to Initial state | -What message should the inspector display? -Does the departure not validate here? | High | true |  |

## 1.4.9 Test Scenario: GATEUYSISCAPII-1083 - Flight Validation 7: Departure flight MVD + flight date NULL

Sumary

| Passenger scans a boarding pass without flight date |  |  |  |  |  |  |  |  |  |
| --- | --- | --- | --- | --- | --- | --- | --- | --- | --- |
| Step | Description | Data | Data 2 | Expected Result | Actual result | To be defined | Priority High, Medium or low | Passed | Responsible |
| 1 | Passenger is instructed a boarding pass | Interior Screen: SC0001 |  | System shall: -Instructing passenger to scan BCBP -Display Screen SC0001 -The eGate doors are closed -The eGate semaphore shows a green arrow in the model -The eGate side illumination are green. | The side illumination of the eGate is red. |  | High | False |  |
| 2 | Passenger presents a boarding pass | Interior Screen: SC0002 | Boarding pass without flight date | System shall: -Display screen SC0002 informing passenger the BCBP is being read -Detect the boarding pass -Barcode reader LED: off | System shall: -Display screen SC0002 informing passenger the BCBP is being read -Detect the boarding pass -Barcode reader LED: off |  |  | true |  |
| 3 | Passenger waits for barcode validation |  |  | System shall: -Do a request to Service with Boardingpass Data, in order to have the validation of BCBP -Service receives the request from the system -Service responds to the system with: code, idTransactionEP and message |  |  |  |  |  |
| 4 | Service responds with "code": different from 0 |  |  | System shall: Continue to reject transaction |  |  |  |  |  |
| Reject transaction |  |  |  |  |  |  |  |  |  |
| Step | Description | Data |  | Expected Result | Actual result | To be defined | Priority High, Medium or low | Passed | Responsible |
| 1 | Passenger is instructed to look for assistance | Interior Screen: SC0004 |  | System shall: -Passenger should look for assistance -Display screen SC0004 -Return to Initial state | System shall: -Passenger should look for assistance -Display screen SC0004 -Return to Initial state | -What message should the inspector display? | High | true |  |

## 1.4.10 Test Scenario: GATEUYSISCAPII-1084 - Flight Validation 8: Departure flight not MVD + flight date NULL

Sumary

| Passenger scans a boarding pass non MVD departure and without flight date |  |  |  |  |  |  |  |  |  |
| --- | --- | --- | --- | --- | --- | --- | --- | --- | --- |
| Step | Description | Data | Data 2 | Expected Result | Actual result | To be defined | Priority High, Medium or low | Passed | Responsible |
| 1 | Passenger is instructed a boarding pass | Interior Screen: SC0001 |  | System shall: -Instructing passenger to scan BCBP -Display Screen SC0001 -The eGate doors are closed -The eGate semaphore shows a green arrow in the model -The eGate side illumination are green. | The side illumination of the eGate is red. |  | High | False |  |
| 2 | Passenger presents a boarding pass | Interior Screen: SC0002 | Boarding pass non MVD departure and without flight date | System shall: -Display screen SC0002 informing passenger the BCBP is being read -Detect the boarding pass -Barcode reader LED: off | System shall: -Display screen SC0002 informing passenger the BCBP is being read -Detect the boarding pass -Barcode reader LED: off |  |  | true |  |
| 3 | Passenger waits for barcode validation |  |  | System shall: -Do a request to Service with Boardingpass Data, in order to have the validation of BCBP -Service receives the request from the system -Service responds to the system with: code, idTransactionEP and message |  |  |  |  |  |
| 4 | Service responds with "code": different from 0 |  |  | System shall: Continue to reject transaction |  |  |  |  |  |
| Reject transaction |  |  |  |  |  |  |  |  |  |
| Step | Description | Data |  | Expected Result | Actual result | To be defined | Priority High, Medium or low | Passed | Responsible |
| 1 | Passenger is instructed to look for assistance | Interior Screen: SC0004 |  | System shall: -Passenger should look for assistance -Display screen SC0004 -Return to Initial state | System shall: -Passenger should look for assistance -Display screen SC0004 -Return to Initial state | -What message should the inspector display? -Does the departure not validate here? | High | true |  |

## 1.4.11 Test Scenario: GATEUYSISCAPII-1085 - Flight Validation 9: Departure NULL+ flight date today

Sumary

| Passenger scans a Boarding Pass NULL departure for the current day |  |  |  |  |  |  |  |  |  |
| --- | --- | --- | --- | --- | --- | --- | --- | --- | --- |
| Step | Description | Data | Data 2 | Expected Result | Actual result | To be defined | Priority High, Medium or low | Passed | Responsible |
| 1 | Passenger is instructed a boarding pass | Interior Screen: SC0001 |  | System shall: -Instructing passenger to scan BCBP -Display Screen SC0001 -The eGate doors are closed -The eGate semaphore shows a green arrow in the model -The eGate side illumination are green. | The side illumination of the eGate is red. |  | High | False |  |
| 2 | Passenger presents a boarding pass | Interior Screen: SC0002 | Boarding Pass NULL departure for the current day | System shall: -Display screen SC0002 informing passenger the BCBP is being read -Detect the boarding pass -Barcode reader LED: off | System shall: -Display screen SC0002 informing passenger the BCBP is being read -Detect the boarding pass -Barcode reader LED: off |  |  | true |  |
| 3 | Passenger waits for barcode validation |  |  | System shall: -Do a request to Service with Boardingpass Data, in order to have the validation of BCBP -Service receives the request from the system -Service responds to the system with: code, idTransactionEP and message |  |  |  |  |  |
| 4 | Service responds with "code": different from 0 |  |  | System shall: Continue to reject transaction |  |  |  |  |  |
| Reject transaction |  |  |  |  |  |  |  |  |  |
| Step | Description | Data |  | Expected Result | Actual result | To be defined | Priority High, Medium or low | Passed | Responsible |
| 1 | Passenger is instructed to look for assistance | Interior Screen: SC0004 |  | System shall: -Passenger should look for assistance -Display screen SC0004 -Return to Initial state | System shall: -Instruct passenger to cross doors -Open doors -Display screen SC0005 |  | High | false |  |
| 2 |  |  |  |  | System shall: -Close doors -Display screen SC0006 -Return to initial state | What message should the inspector display? | High |  |  |

## 1.4.12 Test Scenario: GATEUYSISCAPII-1086 - Flight Validation 10: Departure NULL+ flight date tomorrow

Sumary

| Passenger scans a Boarding Pass NULL departure for the day after |  |  |  |  |  |  |  |  |  |
| --- | --- | --- | --- | --- | --- | --- | --- | --- | --- |
| Step | Description | Data | Data 2 | Expected Result | Actual result | To be defined | Priority High, Medium or low | Passed | Responsible |
| 1 | Passenger is instructed a boarding pass | Interior Screen: SC0001 |  | System shall: -Instructing passenger to scan BCBP -Display Screen SC0001 -The eGate doors are closed -The eGate semaphore shows a green arrow in the model -The eGate side illumination are green. | The side illumination of the eGate is red. |  | High | False |  |
| 2 | Passenger presents a boarding pass | Interior Screen: SC0002 | Boarding Pass NULL departure for the day after | System shall: -Display screen SC0002 informing passenger the BCBP is being read -Detect the boarding pass -Barcode reader LED: off | The system shall: -No SC0002 screen is displayed informing the passenger that the BCBP is being read. -The boarding pass is not detected. |  | High | false | ECSolutions |
| 3 | Passenger waits for barcode validation |  |  | System shall: -Do a request to Service with Boardingpass Data, in order to have the validation of BCBP -Service receives the request from the system -Service responds to the system with: code, idTransactionEP and message |  |  |  |  |  |
| 4 | Service responds with "code": different from 0 |  |  | System shall: Continue to reject transaction |  |  |  |  |  |
| Reject transaction |  |  |  |  |  |  |  |  |  |
| Step | Description | Data |  | Expected Result | Actual result | To be defined | Priority High, Medium or low | Passed | Responsible |
| 1 | Passenger is instructed to look for assistance | Interior Screen: SC0004 |  | System shall: -Passenger should look for assistance -Display screen SC0004 -Return to Initial state | System shall: -Passenger should look for assistance -SC0004 screen is not displayed -Return to Initial state | -What message should the inspector display? -Does the departure not validate here? | High | false |  |

## 1.4.13 Test Scenario: GATEUYSISCAPII-1087 - Flight Validation 11: Departure NULL+ flight date yesterday

Sumary

| Passenger scans a Boarding Pass NULL departure for the day before |  |  |  |  |  |  |  |  |  |
| --- | --- | --- | --- | --- | --- | --- | --- | --- | --- |
| Step | Description | Data | Data 2 | Expected Result | Actual result | To be defined | Priority High, Medium or low | Passed | Responsible |
| 1 | Passenger is instructed a boarding pass | Interior Screen: SC0001 |  | System shall: -Instructing passenger to scan BCBP -Display Screen SC0001 -The eGate doors are closed -The eGate semaphore shows a green arrow in the model -The eGate side illumination are green. | The side illumination of the eGate is red. |  | High | False |  |
| 2 | Passenger presents a boarding pass | Interior Screen: SC0002 | Boarding Pass NULL departure for the day before | System shall: -Display screen SC0002 informing passenger the BCBP is being read -Detect the boarding pass -Barcode reader LED: off | System shall: -Display screen SC0002 informing passenger the BCBP is being read -Detect the boarding pass -Barcode reader LED: off |  |  | true |  |
| 3 | Passenger waits for barcode validation |  |  | System shall: -Do a request to Service with Boardingpass Data, in order to have the validation of BCBP -Service receives the request from the system -Service responds to the system with: code, idTransactionEP and message |  |  |  |  |  |
| 4 | Service responds with "code": different from 0 |  |  | System shall: Continue to reject transaction |  |  |  |  |  |
| Reject transaction |  |  |  |  |  |  |  |  |  |
| Step | Description | Data |  | Expected Result | Actual result | To be defined | Priority High, Medium or low | Passed | Responsible |
| 1 | Passenger is instructed to look for assistance | Interior Screen: SC0004 |  | System shall: -Passenger should look for assistance -Display screen SC0004 -Return to Initial state | System shall: -Passenger should look for assistance -Display screen SC0004 -Return to Initial state | -What message should the inspector display? -Does the departure not validate here? | High | true |  |

## 1.4.14 Test Scenario: GATEUYSISCAPII-1088 - Flight Validation 12: Departure NULL+ flight date NULL

Sumary

| Passenger scans a Boarding Pass NULL departure and flight date NULL |  |  |  |  |  |  |  |  |  |
| --- | --- | --- | --- | --- | --- | --- | --- | --- | --- |
| Step | Description | Data | Data 2 | Expected Result | Actual result | To be defined | Priority High, Medium or low | Passed | Responsible |
| 1 | Passenger is instructed a boarding pass | Interior Screen: SC0001 |  | System shall: -Instructing passenger to scan BCBP -Display Screen SC0001 -The eGate doors are closed -The eGate semaphore shows a green arrow in the model -The eGate side illumination are green. | The side illumination of the eGate is red. |  | High | False |  |
| 2 | Passenger presents a boarding pass | Interior Screen: SC0002 | Boarding Pass NULL departure and flight date NULL | System shall: -Display screen SC0002 informing passenger the BCBP is being read -Detect the boarding pass -Barcode reader LED: off | System shall: -Display screen SC0002 informing passenger the BCBP is being read -Detect the boarding pass -Barcode reader LED: off |  |  | true |  |
| 3 | Passenger waits for barcode validation |  |  | System shall: -Do a request to Service with Boardingpass Data, in order to have the validation of BCBP -Service receives the request from the system -Service responds to the system with: code, idTransactionEP and message |  |  |  |  |  |
| 4 | Service responds with "code": different from 0 |  |  | System shall: Continue to reject transaction |  |  |  |  |  |
| Reject transaction |  |  |  |  |  |  |  |  |  |
| Step | Description | Data |  | Expected Result | Actual result | To be defined | Priority High, Medium or low | Passed | Responsible |
| 1 | Passenger is instructed to look for assistance | Interior Screen: SC0004 |  | System shall: -Passenger should look for assistance -Display screen SC0004 -Return to Initial state | System shall: -Passenger should look for assistance -Display screen SC0004 -Return to Initial state | -What message should the inspector display? -Does the departure not validate here? | High | true |  |

## 1.4.16 Test Scenario: GATEUYSISCAPII-1090 - Gate alarm - Tailgate

Sumary

| Passenger scans a Boarding Pass for the current date |  |  |  |  |  |  |  |  |  |
| --- | --- | --- | --- | --- | --- | --- | --- | --- | --- |
| Step | Description | Data | Data 2 | Expected Result | Actual result | To be defined | Priority High, Medium or low | Passed | Responsible |
| 1 | Passenger is instructed a boarding pass | Interior Screen: SC0001 |  | System shall: -Instructing passenger to scan BCBP -Display Screen SC0001 -The eGate doors are closed -The eGate semaphore shows a green arrow in the model -The eGate side illumination are green. | The side illumination of the eGate is red. |  | High | False |  |
| 2 | Passenger presents a boarding pass | Interior Screen: SC0002 | Boarding Pass for the current date | System shall: -Display screen SC0002 informing passenger the BCBP is being read -Detect the boarding pass -Barcode reader LED: off | System shall: -Display screen SC0002 informing passenger the BCBP is being read -Detect the boarding pass -Barcode reader LED: off |  |  | true |  |
| 3 | Passenger waits for barcode validation |  |  | System shall: -Do a request to Service with Boardingpass Data, in order to have the validation of BCBP -Service receives the request from the system -Service responds to the system with: code, idTransactionEP and message |  |  |  |  |  |
| 4 | Service responds with "code": 0 | { "code": 0, "idTransactionEP": 0, "message": "Boarding correcto" } |  | System shall: Continue to reject transaction |  |  |  |  |  |
| Alarm Tailgate Passenger crosses doors with another passenger |  |  |  |  |  |  |  |  |  |
| Step | Description | Data | Data 2 | Expected Result | Actual result | To be defined | Priority High, Medium or low | Passed | Responsible |
| 1 | Two passengers cross the doors | Interior Screen: SC0005 |  | System Shall: -Display Screen SC0005 -The eGate doors are open -Display in Inspector Tailgate Icon and message -Continue to Reject transaction | System Shall: -Display Screen SC0005 -The eGate doors are open -Two passengers cross the doors -Display in Inspector Tailgate Icon and message -Continue to Reject transaction |  |  | true |  |
| Reject transaction |  |  |  |  |  |  |  |  |  |
| Step | Description | Data |  | Expected Result | Actual result | To be defined | Priority High, Medium or low | Passed | Responsible |
| 1 | Passenger is instructed to look for assistance | Interior Screen: SC0004 |  | System shall: -Passenger should look for assistance -Display screen SC0004 -Return to Initial state | -Passenger is not prompted to seek assistance. -Screen SC0004 is not displayed -Return to Initial state | What message should the inspector display? |  | false |  |

## 1.4.17 Test Scenario: GATEUYSISCAPII-1091 - Gate alarm - Doors Blocked

Sumary

| Passenger scans a Boarding Pass for the current date |  |  |  |  |  |  |  |  |  |
| --- | --- | --- | --- | --- | --- | --- | --- | --- | --- |
| Step | Description | Data | Data 2 | Expected Result | Actual result | To be defined | Priority High, Medium or low | Passed | Responsible |
| 1 | Passenger is instructed a boarding pass | Interior Screen: SC0001 |  | System shall: -Instructing passenger to scan BCBP -Display Screen SC0001 -The eGate doors are closed -The eGate semaphore shows a green arrow in the model -The eGate side illumination are green. | The side illumination of the eGate is red. |  | High | False |  |
| 2 | Passenger presents a boarding pass | Interior Screen: SC0002 | Boarding Pass for the current date | System shall: -Display screen SC0002 informing passenger the BCBP is being read -Detect the boarding pass -Barcode reader LED: off | System shall: -Display screen SC0002 informing passenger the BCBP is being read -Detect the boarding pass -Barcode reader LED: off |  |  | true |  |
| 3 | Passenger waits for barcode validation |  |  | System shall: -Do a request to Service with Boardingpass Data, in order to have the validation of BCBP -Service receives the request from the system -Service responds to the system with: code, idTransactionEP and message |  |  |  |  |  |
| 4 | Service responds with "code": 0 | { "code": 0, "idTransactionEP": 0, "message": "Boarding correcto" } |  | System shall: System shall: -Continue to accept transaction |  |  |  |  |  |
| Alarm Passenger Block Doors |  |  |  |  |  |  |  |  |  |
| Step | Description | Data | Data 2 | Expected Result | Actual result | To be defined | Priority High, Medium or low | Passed | Responsible |
| 1 | Passenger triggers Block Doors Alarm | Interior Screen: SC0005 |  | System Shall: -Display Screen SC0005 to notify passenger -The eGate doors are open -Display in Inspector Blocked doors icon alarm and message -Continue to Accept transaction | System Shall: -Display Screen SC0005 to notify passenger -The eGate doors are open -Passenger crosses through the doors and stands where the sensors are -Display in Inspector Blocked doors icon alarm and message -Continue to Accept transaction | What message should the inspector display? | Medium | true |  |
| Accept transaction |  |  |  |  |  |  |  |  |  |
| Step | Description | Data |  | Expected Result | Actual result | To be defined | Priority High, Medium or low | Passed | Responsible |
| 1 | Passenger is instructed to cross doors | Interior Screen: SC0005 |  | System shall: - Instruct passenger to cross doors -Open doors -Display screen SC0005 | System shall: -Instruct passenger to cross doors -Open doors -Display screen SC0005 |  |  | true |  |
| 2 | Passenger cross doors |  |  | System shall: -Close doors -Return to initial state | System shall: -Close doors -Return to initial state |  |  | true |  |

## 1.4.18 Test Scenario: GATEUYSISCAPII-1092 - Gate alarm - Forced doors

Sumary

| Passenger Forced Doors |  |  |  |  |  |  |  |  |  |
| --- | --- | --- | --- | --- | --- | --- | --- | --- | --- |
| Step | Description | Data | Data 2 | Expected Result | Actual result | To be defined | Priority High, Medium or low | Passed | Responsible |
| 1 | Passenger triggers Forced Doors alarm | Interior Screen: SC0001 |  | System Shall: -Display Screen SC0001 to notify passenger -The eGate are closed -Display in Inspector Forced doors icon alarm and message | System Shall: -Display Screen SC0001 to notify passenger -The eGate are closed -The passenger forces the doors -Display in Inspector Forced doors icon alarm and message | What message should the inspector display? | Medium | true |  |

## 1.4.19 Test Scenario: GATEUYSISCAPII-1093 - Gate alarm - No one detected

Sumary

| Passenger scans a Boarding Pass for the current date |  |  |  |  |  |  |  |  |  |
| --- | --- | --- | --- | --- | --- | --- | --- | --- | --- |
| Step | Description | Data | Data 2 | Expected Result | Actual result | To be defined | Priority High, Medium or low | Passed | Responsible |
| 1 | Passenger is instructed a boarding pass | Interior Screen: SC0001 |  | System shall: -Instructing passenger to scan BCBP -Display Screen SC0001 -The eGate doors are closed -The eGate semaphore shows a green arrow in the model -The eGate side illumination are green. | The side illumination of the eGate is red. |  | High | False |  |
| 2 | Passenger presents a boarding pass | Interior Screen: SC0002 | valid boarding pass | System shall: -Display screen SC0002 informing passenger the BCBP is being read -Detect the boarding pass -Barcode reader LED: off |  | -Should the barcode reader's LED light green when it detects boarding? | High | true |  |
| 3 | Passenger waits for barcode validation |  |  | System shall: -Do a request to Service with Boardingpass Data, in order to have the validation of BCBP -Service receives the request from the system -Service responds to the system with: code, idTransactionEP and message |  |  |  |  |  |
| 4 | Service responds with "code": 0 | { "code": 0 "idTransactionEP": 0, "message": "Boarding correcto" } |  | System shall: Continue to accept transaction |  |  |  |  |  |
| Accept transaction |  |  |  |  |  |  |  |  |  |
| Step | Description | Data |  | Expected Result | Actual result | To be defined | Priority High, Medium or low | Passed | Responsible |
| 1 | Passenger is instructed to cross doors | Interior Screen: SC0005 |  | System shall: -Instruct passenger to cross doors -Open doors -Display screen SC0005 | System shall: -Instruct passenger to cross doors -Open doors -Display screen SC0005 |  |  | true |  |
| 2 | Passenger does not cross the doors until timeout |  |  | System shall: -Close doors -Return to initial state | System shall: -Close doors -Return to initial state | What alarm and message should the inspector display? | High | true |  |

# 1.4.20 Other test cases

## 1.4.20.1 Open and close the eGate doors from the inspector.

Sumary

| Open and close the eGate doors from the inspector |  |  |  |  |  |  |  |  |  |
| --- | --- | --- | --- | --- | --- | --- | --- | --- | --- |
| Step | Description | Data | Data 2 | Expected Result | Actual result | To be defined | Priority High, Medium or low | Passed | Responsible |
| 1 | Opening and closing the eGate doors takes too long from the inspector. |  |  |  |  | How long does it take to open and close the eGate doors from the inspector? | low |  | Vision-Box |

## 1.4.20.2 Opening the eGate doors from the inspector.

Sumary

| Open the eGate doors from the inspector |  |  |  |  |  |  |  |  |  |
| --- | --- | --- | --- | --- | --- | --- | --- | --- | --- |
| Step | Description | Data | Data 2 | Expected Result | Actual result | To be defined | Priority High, Medium or low | Passed | Responsible |
| 1 | The eGate doors are closed and the inspector displays the Close All Doors option instead of Open All Doors. See Annex A. |  | The eGate doors are closed. | The inspector displays the Open All Doors option. | The inspector displays the Close all doors option |  | low | false | Vision-Box |

## 1.4.20.3 Restarting the gateway from the inspector

Sumary

| Restarting the gateway from the inspector |  |  |  |  |  |  |  |  |  |
| --- | --- | --- | --- | --- | --- | --- | --- | --- | --- |
| Step | Description | Data | Data 2 | Expected Result | Actual result | To be defined | Priority High, Medium or low | Passed | Responsible |
| 1 | When the gateway is restarted from the inspector it is initially shown out of service. |  |  |  |  | What screen should the gateway display when restarting from the inspector? | low |  | Vision-Box |

## 1.4.20.4 Abandoned object alarm

Sumary

| Passenger scans a Boarding Pass for the current date |  |  |  |  |  |  |  |  |  |
| --- | --- | --- | --- | --- | --- | --- | --- | --- | --- |
| Step | Description | Data | Data 2 | Expected Result | Actual result | To be defined | Priority High, Medium or low | Passed | Responsible |
| 1 | Passenger is instructed a boarding pass | Interior Screen: SC0001 |  | System shall: -Instructing passenger to scan BCBP -Display Screen SC0001 -The eGate doors are closed -The eGate semaphore shows a green arrow in the model -The eGate side illumination are green. | System shall: -Instructing passenger to scan BCBP -Display Screen SC0001 -The eGate doors are closed -The eGate semaphore shows a green arrow in the model - The eGate side illumination are red. |  | High | False |  |
| 2 | Passenger presents a boarding pass | Interior Screen: SC0002 | valid boarding pass | System shall: -Display screen SC0002 informing passenger the BCBP is being read -Detect the boarding pass -Barcode reader LED: off | System shall: -Display screen SC0002 informing passenger the BCBP is being read -Detect the boarding pass -Barcode reader LED: off | -Should the barcode reader's LED light green when it detects boarding? | High | true |  |
| 3 | Passenger waits for barcode validation |  |  | System shall: -Do a request to Service with Boardingpass Data, in order to have the validation of BCBP -Service receives the request from the system -Service responds to the system with: code, idTransactionEP and message |  |  |  |  |  |
| 4 | Service responds with "code": 0 | { "code": 0 "idTransactionEP": 0, "message": "Boarding correcto" } |  | System shall: Continue to accept transaction |  |  |  |  |  |
| Accept transaction |  |  |  |  |  |  |  |  |  |
| Step | Description | Data |  | Expected Result | Actual result | To be defined | Priority High, Medium or low | Passed | Responsible |
| 1 | Passenger is instructed to cross doors | Interior Screen: SC0005 |  | System shall: -Instruct passenger to cross doors -Open doors -Display screen SC0005 | System shall: -Instruct passenger to cross doors -Open doors -Display screen SC0005 |  |  | true |  |
| 2 | Passenger cross doors |  |  |  | System shall: - Passenger through the doors leaving an object behind -Close doors -Display screen SC0006 -Return to initial state | -Define expected result in case of object abandonment by passenger -What alarm and message should the inspector display when faced with an abandoned object? | High | false |  |

## 1.4.20.5 Person standing on the other side of the door where the sensor is located

Sumary

| The passenger scans a boarding pass for the current date with a passenger on the other side of the door next to the sensor. |  |  |  |  |  |  |  |  |  |
| --- | --- | --- | --- | --- | --- | --- | --- | --- | --- |
| Step | Description | Data | Data 2 | Expected Result | Actual result | To be defined | Priority High, Medium or low | Passed | Responsible |
| 1 | Passenger is instructed a boarding pass |  |  | System shall: -Instructing passenger to scan BCBP -Display Screen SC0001 -The eGate doors are closed -The eGate semaphore shows a green arrow in the model -The eGate side illumination are green. | System shall: -Instructing passenger to scan BCBP -Display Screen SC0001 -The eGate doors are closed -The eGate semaphore shows a green arrow in the model - The eGate side illumination are red. |  | High | false | Vision-Box |
| 2 | The passenger scans a boarding pass for the current date with a passenger on the other side of the door next to the sensor. |  | Valid boarding pass |  | System shall: -Display screen SC0002 informing passenger the BCBP is being read -Detect the boarding pass -Barcode reader LED: off -Instruct passenger to cross doors -Open doors -Display screen SC0005 -Passenger walks through the door -Close doors -Display screen SC0006 -Return to initial state | What is the expected result with a passenger on the other side of the door, next to the sensor? | High |  | Vision-Box |

## 1.4.20.6 NULL flight destination + date of today's flight

Sumary

| Passenger scans a boarding pass Null destination for the current day |  |  |  |  |  |  |  |  |  |
| --- | --- | --- | --- | --- | --- | --- | --- | --- | --- |
| Step | Description | Data | Data 2 | Expected Result | Actual result | To be defined | Priority High, Medium or low | Passed | Responsible |
| 1 | Passenger is instructed a boarding pass | Interior Screen: SC0001 |  | System shall: -Instructing passenger to scan BCBP -Display Screen SC0001 -The eGate doors are closed -The eGate semaphore shows a green arrow in the model -The eGate side illumination are green. | System shall: -Instructing passenger to scan BCBP -Display Screen SC0001 -The eGate doors are closed -The eGate semaphore shows a green arrow in the model -The eGate side illumination are red. |  | High | False | Vision-Box |
| 2 | Passenger presents a boarding pass Null destination |  | Boarding pass Null destination |  | -Display screen SC0002 informing passenger the BCBP is being read -Detect the boarding pass -Barcode reader LED: off -Instruct passenger to cross doors -Open doors -Display screen SC0005 -Passenger walks through the door -Close doors -Display screen SC0006 -Return to initial state | What is the expected result for a null destination? | High |  | Vision-Box |

## 1.4.20.7 Numeric flight destination + today's flight date

Sumary

| The passenger scans a boarding pass with a numerical destination value for the current day. |  |  |  |  |  |  |  |  |  |
| --- | --- | --- | --- | --- | --- | --- | --- | --- | --- |
| Step | Description | Data | Data 2 | Expected Result | Actual result | To be defined | Priority High, Medium or low | Passed | Responsible |
| 1 | Passenger is instructed a boarding pass | Interior Screen: SC0001 |  | System shall: -Instructing passenger to scan BCBP -Display Screen SC0001 -The eGate doors are closed -The eGate semaphore shows a green arrow in the model -The eGate side illumination are green. | System shall: -Instructing passenger to scan BCBP -Display Screen SC0001 -The eGate doors are closed -The eGate semaphore shows a green arrow in the model -The eGate side illumination are red. |  | High | False | Vision-Box |
| 2 | Passenger presents a boarding pass Numeric Destination |  | Boarding pass Numerical destination |  | -Display screen SC0002 informing passenger the BCBP is being read -Detect the boarding pass -Barcode reader LED: off -Instruct passenger to cross doors -Open doors -Display screen SC0005 -Passenger walks through the door -Close doors -Display screen SC0006 -Return to initial state | What is the expected result for a numerical destination? | High |  | Vision-Box |

## 1.4.20.8 Flight destination consisting of special characters + today's flight date

Sumary

| The passenger scans a boarding pass with a value equal to special characters as the destination for the current day. |  |  |  |  |  |  |  |  |  |
| --- | --- | --- | --- | --- | --- | --- | --- | --- | --- |
| Step | Description | Data | Data 2 | Expected Result | Actual result | To be defined | Priority High, Medium or low | Passed | Responsible |
| 1 | Passenger is instructed a boarding pass | Interior Screen: SC0001 |  | System shall: -Instructing passenger to scan BCBP -Display Screen SC0001 -The eGate doors are closed -The eGate semaphore shows a green arrow in the model -The eGate side illumination are green. | System shall: -Instructing passenger to scan BCBP -Display Screen SC0001 -The eGate doors are closed -The eGate semaphore shows a green arrow in the model -The eGate side illumination are red. |  | High | False | Vision-Box |
| 2 | The passenger presents a boarding pass with a value equal to special characters as destination. |  | Boarding pass with a special character value as destination |  | -Display screen SC0002 informing passenger the BCBP is being read -Detect the boarding pass -Barcode reader LED: off -Instruct passenger to cross doors -Open doors -Display screen SC0005 -Passenger walks through the door -Close doors -Display screen SC0006 -Return to initial state | - What is the expected result for a destination with character value? -What message should the inspector display? | High |  | Vision-Box |

## 1.4.20.9 Destination consisting of four characters+ date of today's flight

Sumary

| Passenger scans a boarding pass with a destination consisting of four characters |  |  |  |  |  |  |  |  |  |
| --- | --- | --- | --- | --- | --- | --- | --- | --- | --- |
| Step | Description | Data | Data 2 | Expected Result | Actual result | To be defined | Priority High, Medium or low | Passed | Responsible |
| 1 | Passenger is instructed a boarding pass | Interior Screen: SC0001 |  | System shall: -Instructing passenger to scan BCBP -Display Screen SC0001 -The eGate doors are closed -The eGate semaphore shows a green arrow in the model -The eGate side illumination are green. | System shall: -Instructing passenger to scan BCBP -Display Screen SC0001 -The eGate doors are closed -The eGate semaphore shows a green arrow in the model -The eGate side illumination are red. |  | High | False | Vision-Box |
| 2 | Passenger presents a boarding pass with a destination consisting of four characters |  | Boarding pass with a destination consisting of four characters |  | -Display screen SC0002 informing passenger the BCBP is being read -Detect the boarding pass -Barcode reader LED: off -Passenger should look for assistance -Display screen SC0004 -Return to Initial state | -What is the expected result for a destination consisting of four characters? -What message should the inspector display? | High |  | Vision-Box |

## 1.4.20.10 Destination composed of two characters + today's flight date

Sumary

| Passenger scans a boarding pass with a destination consisting of two characters |  |  |  |  |  |  |  |  |  |
| --- | --- | --- | --- | --- | --- | --- | --- | --- | --- |
| Step | Description | Data | Data 2 | Expected Result | Actual result | To be defined | Priority High, Medium or low | Passed | Responsible |
| 1 | Passenger is instructed a boarding pass | Interior Screen: SC0001 |  | System shall: -Instructing passenger to scan BCBP -Display Screen SC0001 -The eGate doors are closed -The eGate semaphore shows a green arrow in the model -The eGate side illumination are green. | System shall: -Instructing passenger to scan BCBP -Display Screen SC0001 -The eGate doors are closed -The eGate semaphore shows a green arrow in the model -The eGate side illumination are red. |  | High | False | Vision-Box |
| 2 | Passenger presents a boarding pass with a destination consisting of two characters |  | Boarding pass with a destination consisting of two characters |  | -Display screen SC0002 informing passenger the BCBP is being read -Detect the boarding pass -Barcode reader LED: off -Instruct passenger to cross doors -Open doors -Display screen SC0005 -Passenger walks through the door -Close doors -Display screen SC0006 -Return to initial state | -What is the expected result for a destination consisting of two characters? -What message should the inspector display? | High |  | Vision-Box |

## 1.4.20.11 Origin composed of two characters + today's flight date

Sumary

| The passenger scans a boarding pass with an origin consisting of two characters |  |  |  |  |  |  |  |  |  |
| --- | --- | --- | --- | --- | --- | --- | --- | --- | --- |
| Step | Description | Data | Data 2 | Expected Result | Actual result | To be defined | Priority High, Medium or low | Passed | Responsible |
| 1 | Passenger is instructed a boarding pass | Interior Screen: SC0001 |  | System shall: -Instructing passenger to scan BCBP -Display Screen SC0001 -The eGate doors are closed -The eGate semaphore shows a green arrow in the model -The eGate side illumination are green. | System shall: -Instructing passenger to scan BCBP -Display Screen SC0001 -The eGate doors are closed -The eGate semaphore shows a green arrow in the model -The eGate side illumination are red. |  | High | False | Vision-Box |
| 2 | The passenger presents a boarding pass with an origin consisting of two characters |  | Boarding pass with an origin consisting of two characters |  | -Display screen SC0002 informing passenger the BCBP is being read -Detect the boarding pass -Barcode reader LED: off -Instruct passenger to cross doors -Open doors -Display screen SC0005 -Passenger walks through the door -Close doors -Display screen SC0006 -Return to initial state | -What is the expected result for an origin consisting of two characters? -What message should the inspector display? | High |  | Vision-Box |

## 1.4.20.12 Origin composed of four characters + today's flight date

Sumary

| The passenger scans a boarding pass whose origin consists of four characters |  |  |  |  |  |  |  |  |  |
| --- | --- | --- | --- | --- | --- | --- | --- | --- | --- |
| Step | Description | Data | Data 2 | Expected Result | Actual result | To be defined | Priority High, Medium or low | Passed | Responsible |
| 1 | Passenger is instructed a boarding pass | Interior Screen: SC0001 |  | System shall: -Instructing passenger to scan BCBP -Display Screen SC0001 -The eGate doors are closed -The eGate semaphore shows a green arrow in the model -The eGate side illumination are green. | System shall: -Instructing passenger to scan BCBP -Display Screen SC0001 -The eGate doors are closed -The eGate semaphore shows a green arrow in the model -The eGate side illumination are red. |  | High | False | Vision-Box |
| 2 | The passenger presents a boarding pass whose origin consists of four characters |  | Boarding pass with an origin consisting of four characters |  | -Display screen SC0002 informing passenger the BCBP is being read -Detect the boarding pass -Barcode reader LED: off -Passenger should look for assistance -Display screen SC0004 -Return to Initial state | -What is the expected result for an origin consisting of four characters? -What message should the inspector display? | High |  | Vision-Box |

## 1.4.20.13 Origin formed by numbers + today's flight date

Sumary

| The passenger scans a boarding pass with a numerical origin |  |  |  |  |  |  |  |  |  |
| --- | --- | --- | --- | --- | --- | --- | --- | --- | --- |
| Step | Description | Data | Data 2 | Expected Result | Actual result | To be defined | Priority High, Medium or low | Passed | Responsible |
| 1 | Passenger is instructed a boarding pass | Interior Screen: SC0001 |  | System shall: -Instructing passenger to scan BCBP -Display Screen SC0001 -The eGate doors are closed -The eGate semaphore shows a green arrow in the model -The eGate side illumination are green. | System shall: -Instructing passenger to scan BCBP -Display Screen SC0001 -The eGate doors are closed -The eGate semaphore shows a green arrow in the model -The eGate side illumination are red. |  | High | False | Vision-Box |
| 2 | The passenger presents a boarding pass with a numerical origin. |  | Boarding pass with a numerical origin |  | -Display screen SC0002 informing passenger the BCBP is being read -Detect the boarding pass -Barcode reader LED: off -Instruct passenger to cross doors -Open doors -Display screen SC0005 -Passenger walks through the door -Close doors -Display screen SC0006 -Return to initial state | -What is the expected result for a numerical origin? -What message should the inspector display? | High |  | Vision-Box |

## 1.4.20.14 Origin consisting of special characters+ today's flight date

Sumary

| Passenger scans a boarding pass with an origin consisting of special characters |  |  |  |  |  |  |  |  |  |
| --- | --- | --- | --- | --- | --- | --- | --- | --- | --- |
| Step | Description | Data | Data 2 | Expected Result | Actual result | To be defined | Priority High, Medium or low | Passed | Responsible |
| 1 | Passenger is instructed a boarding pass | Interior Screen: SC0001 |  | System shall: -Instructing passenger to scan BCBP -Display Screen SC0001 -The eGate doors are closed -The eGate semaphore shows a green arrow in the model -The eGate side illumination are green. | System shall: -Instructing passenger to scan BCBP -Display Screen SC0001 -The eGate doors are closed -The eGate semaphore shows a green arrow in the model -The eGate side illumination are red. |  | High | False | Vision-Box |
| 2 | The passenger presents a boarding pass with an origin consisting of special characters. |  | Boarding pass with origin consisting of special characters |  | -Display screen SC0002 informing passenger the BCBP is being read -Detect the boarding pass -Barcode reader LED: off -Instruct passenger to cross doors -Open doors -Display screen SC0005 -Passenger walks through the door -Close doors -Display screen SC0006 -Return to initial state | -What is the expected result for a origin consisting of special characters? -What message should the inspector display? | High |  | Vision-Box |

## 1.4.20.15 Flight code null+today's flight date

Sumary

| Passenger scans boarding pass with null flight code |  |  |  |  |  |  |  |  |  |
| --- | --- | --- | --- | --- | --- | --- | --- | --- | --- |
| Step | Description | Data | Data 2 | Expected Result | Actual result | To be defined | Priority High, Medium or low | Passed | Responsible |
| 1 | Passenger is instructed a boarding pass | Interior Screen: SC0001 |  | System shall: -Instructing passenger to scan BCBP -Display Screen SC0001 -The eGate doors are closed -The eGate semaphore shows a green arrow in the model -The eGate side illumination are green. | System shall: -Instructing passenger to scan BCBP -Display Screen SC0001 -The eGate doors are closed -The eGate semaphore shows a green arrow in the model -The eGate side illumination are red. |  | High | False | Vision-Box |
| 2 | Passenger presents a boarding pass with a null flight code |  | Boarding pass with null flight code |  | -Display screen SC0002 informing passenger the BCBP is being read -Detect the boarding pass -Barcode reader LED: off -Passenger should look for assistance -Display screen SC0004 -Return to Initial state | -What is the expected result for a null flight code? -What message should the inspector display? | High |  | Vision-Box |

## 1.4.20.16 Numeric flight code+today's flight date

Sumary

| The passenger scans the boarding pass with the flight code equal to a number |  |  |  |  |  |  |  |  |  |
| --- | --- | --- | --- | --- | --- | --- | --- | --- | --- |
| Step | Description | Data | Data 2 | Expected Result | Actual result | To be defined | Priority High, Medium or low | Passed | Responsible |
| 1 | Passenger is instructed a boarding pass | Interior Screen: SC0001 |  | System shall: -Instructing passenger to scan BCBP -Display Screen SC0001 -The eGate doors are closed -The eGate semaphore shows a green arrow in the model -The eGate side illumination are green. | System shall: -Instructing passenger to scan BCBP -Display Screen SC0001 -The eGate doors are closed -The eGate semaphore shows a green arrow in the model -The eGate side illumination are red. |  | High | False | Vision-Box |
| 2 | The passenger presents a boarding pass with a numerical flight code. |  | Boarding pass with a numerical flight code. |  | -Display screen SC0002 informing passenger the BCBP is being read -Detect the boarding pass -Barcode reader LED: off -Passenger should look for assistance -Display screen SC0004 -Return to Initial state | -What is the expected result for a numerical flight code? -What message should the inspector display? | High |  | Vision-Box |

## 1.4.20.17 Alphabetical flight code + today's flight date

Sumary

| Passenger scans the boarding pass with the alphabetical flight code |  |  |  |  |  |  |  |  |  |
| --- | --- | --- | --- | --- | --- | --- | --- | --- | --- |
| Step | Description | Data | Data 2 | Expected Result | Actual result | To be defined | Priority High, Medium or low | Passed | Responsible |
| 1 | Passenger is instructed a boarding pass | Interior Screen: SC0001 |  | System shall: -Instructing passenger to scan BCBP -Display Screen SC0001 -The eGate doors are closed -The eGate semaphore shows a green arrow in the model -The eGate side illumination are green. | System shall: -Instructing passenger to scan BCBP -Display Screen SC0001 -The eGate doors are closed -The eGate semaphore shows a green arrow in the model -The eGate side illumination are red. |  | High | False | Vision-Box |
| 2 | The passenger presents a boarding pass with an alphabetical flight code. |  | Boarding pass with alphabetical flight code. |  | -Display screen SC0002 informing passenger the BCBP is being read -Detect the boarding pass -Barcode reader LED: off -Passenger should look for assistance -Display screen SC0004 -Return to Initial state | -What is the expected result for an alphabetic flight code? -What message should the inspector display? | High |  | Vision-Box |

## 1.4.20.18 Flight code made up of special characters + date of today's flight

Sumary

| The passenger scans the boarding pass with the flight code made up of special characters. |  |  |  |  |  |  |  |  |  |
| --- | --- | --- | --- | --- | --- | --- | --- | --- | --- |
| Step | Description | Data | Data 2 | Expected Result | Actual result | To be defined | Priority High, Medium or low | Passed | Responsible |
| 1 | Passenger is instructed a boarding pass | Interior Screen: SC0001 |  | System shall: -Instructing passenger to scan BCBP -Display Screen SC0001 -The eGate doors are closed -The eGate semaphore shows a green arrow in the model -The eGate side illumination are green. | System shall: -Instructing passenger to scan BCBP -Display Screen SC0001 -The eGate doors are closed -The eGate semaphore shows a green arrow in the model -The eGate side illumination are red. |  | High | False | Vision-Box |
| 2 | The passenger presents a boarding pass with a flight code made up of special characters. |  | Boarding pass with flight code made up of special characters. |  | -Display screen SC0002 informing passenger the BCBP is being read -Detect the boarding pass -Barcode reader LED: off -Passenger should look for assistance -Display screen SC0004 -Return to Initial state | -What is the expected result for a flight code consisting of special characters? -What message should the inspector display? | High |  | Vision-Box |

## 1.4.20.19 Scan boarding with flight code under 8 characters

Sumary

| Passenger scans boarding pass with flight code of less than 8 characters |  |  |  |  |  |  |  |  |  |
| --- | --- | --- | --- | --- | --- | --- | --- | --- | --- |
| Step | Description | Data | Data 2 | Expected Result | Actual result | To be defined | Priority High, Medium or low | Passed | Responsible |
| 1 | Passenger is instructed a boarding pass | Interior Screen: SC0001 |  |  | System shall: -Instructing passenger to scan BCBP -Display Screen SC0001 -The eGate doors are closed -The eGate semaphore shows a green arrow in the model -The eGate side illumination are red. |  | High | False | Vision-Box |
| 2 | The passenger presents a boarding pass with a flight code of less than 8 characters. |  | Boarding pass with flight code shorter than 8 characters |  | -Display screen SC0002 informing passenger the BCBP is being read -Detect the boarding pass -Barcode reader LED: off -Passenger should look for assistance -Display screen SC0004 -Return to Initial state | -What is the expected result for a flight code less than 8 characters? -What message should the inspector display? | High |  | Vision-Box |

## 1.4.20.20 Scan boarding with a flight code longer than 8 characters

Sumary

| The passenger scans the boarding pass with a flight code of more than 8 characters. |  |  |  |  |  |  |  |  |  |
| --- | --- | --- | --- | --- | --- | --- | --- | --- | --- |
| Step | Description | Data | Data 2 | Expected Result | Actual result | To be defined | Priority High, Medium or low | Passed | Responsible |
| 1 | Passenger is instructed a boarding pass | Interior Screen: SC0001 |  |  | System shall: -Instructing passenger to scan BCBP -Display Screen SC0001 -The eGate doors are closed -The eGate semaphore shows a green arrow in the model -The eGate side illumination are red. |  | High | False | Vision-Box |
| 2 | The passenger presents a boarding pass with a flight code of more than 8 characters. |  | Boarding pass with flight code longer than 8 characters |  | -Display screen SC0002 informing passenger the BCBP is being read -Detect the boarding pass -Barcode reader LED: off -Passenger should look for assistance -Display screen SC0004 -Return to Initial state | -What is the expected result for a flight code with more than 8 characters? -What message should the inspector display? | High |  | Vision-Box |

## 1.4.20.21 Scan boarding with a flight seat more than 4 characters long

Sumary

| Passenger scans a boarding pass with a flight seat consisting of more than 4 characters |  |  |  |  |  |  |  |  |  |
| --- | --- | --- | --- | --- | --- | --- | --- | --- | --- |
| Step | Description | Data | Data 2 | Expected Result | Actual result | To be defined | Priority High, Medium or low | Passed | Responsible |
| 1 | Passenger is instructed a boarding pass | Interior Screen: SC0001 |  | System shall: -Instructing passenger to scan BCBP -Display Screen SC0001 -The eGate doors are closed -The eGate semaphore shows a green arrow in the model -The eGate side illumination are green. | System shall: -Instructing passenger to scan BCBP -Display Screen SC0001 -The eGate doors are closed -The eGate semaphore shows a green arrow in the model -The eGate side illumination are red. |  | High | False | Vision-Box |
| 2 | The passenger presents a boarding pass with a seat consisting of more than 4 characters. |  | Boarding pass with seat consisting of more than 4 characters |  | -Display screen SC0002 informing passenger the BCBP is being read -Detect the boarding pass -Barcode reader LED: off -Instruct passenger to cross doors -Open doors -Display screen SC0005 -Passenger walks through the door -Close doors -Display screen SC0006 -Return to initial state | -What is the expected result for an seat consisting of more than 4 characters? -What message should the inspector display? | High |  |  |

## 1.4.20.22 Scan boarding with a flight seat less than 4 characters long

Sumary

| The passenger scans the boarding pass with a seat consisting of less than 4 characters. |  |  |  |  |  |  |  |  |  |
| --- | --- | --- | --- | --- | --- | --- | --- | --- | --- |
| Step | Description | Data | Data 2 | Expected Result | Actual result | To be defined | Priority High, Medium or low | Passed | Responsible |
| 1 | Passenger is instructed a boarding pass | Interior Screen: SC0001 |  |  | System shall: -Instructing passenger to scan BCBP -Display Screen SC0001 -The eGate doors are closed -The eGate semaphore shows a green arrow in the model -The eGate side illumination are red. |  | High | False | Vision-Box |
| 2 | Passenger presents a boarding pass for a seat consisting of less than 4 characters. |  | Boarding pass with seat less than 4 characters long |  | -Display screen SC0002 informing passenger the BCBP is being read -Detect the boarding pass -Barcode reader LED: off -Passenger should look for assistance -Display screen SC0004 -Return to Initial state | -What is the expected result for an seat of less than 4 characters? -What message should the inspector display? | High |  | Vision-Box |

## 1.4.20.23 Scan boarding with a seat with alphabetical value

Sumary

| The passenger scans the boarding pass with a seat that has an alphabetical value. |  |  |  |  |  |  |  |  |  |
| --- | --- | --- | --- | --- | --- | --- | --- | --- | --- |
| Step | Description | Data | Data 2 | Expected Result | Actual result | To be defined | Priority High, Medium or low | Passed | Responsible |
| 1 | Passenger is instructed a boarding pass | Interior Screen: SC0001 |  |  | System shall: -Instructing passenger to scan BCBP -Display Screen SC0001 -The eGate doors are closed -The eGate semaphore shows a green arrow in the model -The eGate side illumination are red. |  | High | False | Vision-Box |
| 2 | The passenger presents a boarding pass for a seat with an alphabetical value. |  | Boarding pass with seat formed by alphabetical value |  | -Display screen SC0002 informing passenger the BCBP is being read -Detect the boarding pass -Barcode reader LED: off -Passenger should look for assistance -Display screen SC0004 -Return to Initial state | -What is the expected result for an seat formed by alphabetical value? -What message should the inspector display? | High |  | Vision-Box |

## 1.4.20.24 Scan boarding with a seat with a value equal to special characters

Sumary

| Passenger scans boarding pass with a seat made up of special characters |  |  |  |  |  |  |  |  |  |
| --- | --- | --- | --- | --- | --- | --- | --- | --- | --- |
| Step | Description | Data | Data 2 | Expected Result | Actual result | To be defined | Priority High, Medium or low | Passed | Responsible |
| 1 | Passenger is instructed a boarding pass | Interior Screen: SC0001 |  |  | System shall: -Instructing passenger to scan BCBP -Display Screen SC0001 -The eGate doors are closed -The eGate semaphore shows a green arrow in the model -The eGate side illumination are red. |  | High | False | Vision-Box |
| 2 | Passenger presents a boarding pass for a seat made up of special characters |  | Boarding pass with seat made up of special characters |  | -Display screen SC0002 informing passenger the BCBP is being read -Detect the boarding pass -Barcode reader LED: off -Passenger should look for assistance -Display screen SC0004 -Return to Initial state | -What is the expected result for an seat consisting of special characters? -What message should the inspector display? | High |  | Vision-Box |

## 1.4.20.25 Scan boarding with a seat with a value equal to only numbers

Sumary

| Passenger scans a boarding pass with a seat made up of only numbers |  |  |  |  |  |  |  |  |  |
| --- | --- | --- | --- | --- | --- | --- | --- | --- | --- |
| Step | Description | Data | Data 2 | Expected Result | Actual result | To be defined | Priority High, Medium or low | Passed | Responsible |
| 1 | Passenger is instructed a boarding pass | Interior Screen: SC0001 |  | System shall: -Instructing passenger to scan BCBP -Display Screen SC0001 -The eGate doors are closed -The eGate semaphore shows a green arrow in the model -The eGate side illumination are green. | System shall: -Instructing passenger to scan BCBP -Display Screen SC0001 -The eGate doors are closed -The eGate semaphore shows a green arrow in the model -The eGate side illumination are red. |  | High | False | Vision-Box |
| 2 | The passenger presents a boarding pass with a seat made up of only numbers. |  | Boarding pass with seat made up of only numbers |  | -Display screen SC0002 informing passenger the BCBP is being read -Detect the boarding pass -Barcode reader LED: off -Instruct passenger to cross doors -Open doors -Display screen SC0005 -Passenger walks through the door -Close doors -Display screen SC0006 -Return to initial state | -What is the expected result for an seat consisting of only numbers? -What message should the inspector display? | High |  |  |

## 1.4.20.26 Scan boarding for an already boarded passenger

Sumary

| Passenger scans boarding pass of an already boarded passenger |  |  |  |  |  |  |  |  |  |
| --- | --- | --- | --- | --- | --- | --- | --- | --- | --- |
| Step | Description | Data | Data 2 | Expected Result | Actual result | To be defined | Priority High, Medium or low | Passed | Responsible |
| 1 | Passenger is instructed a boarding pass | Interior Screen: SC0001 |  |  | System shall: -Instructing passenger to scan BCBP -Display Screen SC0001 -The eGate doors are closed -The eGate semaphore shows a green arrow in the model -The eGate side illumination are red. |  | High | False | Vision-Box |
| 2 | The passenger presents a boarding pass of a passenger who has already boarded. |  | Boarding card of a passenger already boarded |  | -Display screen SC0002 informing passenger the BCBP is being read -Detect the boarding pass -Barcode reader LED: off -Passenger should look for assistance -Display screen SC0004 -Return to Initial state | -What is the expected result for a passenger who presents boarding of a passenger already boarded? -What message should the inspector display? | High |  | Vision-Box |

## 1.4.20.27 Scan boarding of a passenger with the same seat as another passenger who does not confirm boarding

Sumary

| The passenger scans the boarding pass with the same seat as a passenger who does not confirm boarding. |  |  |  |  |  |  |  |  |  |
| --- | --- | --- | --- | --- | --- | --- | --- | --- | --- |
| Step | Description | Data | Data 2 | Expected Result | Actual result | To be defined | Priority High, Medium or low | Passed | Responsible |
| 1 | Passenger is instructed a boarding pass | Interior Screen: SC0001 |  |  | System shall: -Instructing passenger to scan BCBP -Display Screen SC0001 -The eGate doors are closed -The eGate semaphore shows a green arrow in the model -The eGate side illumination are red. |  | High | False | Vision-Box |
| 2 | The passenger presents the boarding pass with the same seat as a passenger who does not confirm boarding. |  | Boarding pass with the same seat as a passenger who does not confirm boarding |  | -Display screen SC0002 informing passenger the BCBP is being read -Detect the boarding pass -Barcode reader LED: off -Passenger should look for assistance -Display screen SC0004 -Return to Initial state | - -What is the expected result for a passenger who presents boarding with the same seat as a passenger who does not confirm boarding? -What message should the inspector display? | High |  | Vision-Box |

# 1.4.21 Annexes

## Annex A