GATEUYSISCAPII-887_TestPlan_SCP 23/11/2022

GATE-UY-SISCA-P2 Test Plan

Security Checkpoint Gate Document Date: 23/11/2022

Property Rights

© Copyright 2020, Vision-Box®

COMMERCIAL IN CONFIDENCE www.vision-box.com

1/32

GATEUYSISCAPII-887_TestPlan_SCP 23/11/2022

All rights to this document are reserved. The contents of this document are the intellectual property of Vision-Box®.

Other than user rights contractually granted, the copying, cession to third parties, or transmission of the contents of this document without its prior written consent is, consequently, forbidden. No part of this publication may be reproduced, stored, distributed, transmitted, transcribed, used, or translated into any other language, in any manner or through any means – whether graphical, electronic, or mechanical, and including its photocopying, saving and storage in any data retrieval systems – without the prior written consent of Vision-Box®.

Legal Notice

Unauthorized duplication or usage of the information presented in this document, partially or in full, and in printed format or in any other manner or through any other means, is expressly forbidden.

Exceptas otherwisestatedherein,nothingcontainedinthis document,or infurther documents referencedherein,is tobeconstrued as authority for varying the terms of any agreement or any individual contract generated under any agreement.

Limitation of responsibility

Although this document was reviewed and validated, it may contain typographical or technical errors.

This document is subject to a revision process from time to time, and such revisions shall be included in future issues thereof. Vision-Box®. may proceed to incorporate such modifications whenever required.

Vision-Box®. does not assume any responsibility for damages that may be directly or indirectly caused by errors, omissions or discrepancies which may arise between software applications, equipments and this document.

Licenses and Registered Trademarks

This publication contains references or information about the following products:

Vision-Box®, vision-box SENTRY®, VB-ePASS®, VB-eGate®, VB i-Match®, are registered trademarks of Vision-Box®; Microsoft and Windows are registered trademarks of Microsoft Corporation; PC is a registered trademark of International Business Corporation; Pentium is a registered trademark of Intel Corporation.

Other product and company names may be registered trademarks of other companies and are used exclusively for clarification purposes and user guidance, and without intention of infringing any such rights.

COMMERCIAL IN CONFIDENCE www.vision-box.com

2/32

GATEUYSISCAPII-887_TestPlan_SCP 23/11/2022

Test Plan: Security Checkpoint Gate

Version	Date	Changed by	Approved by	Change Details

1.0.1	08/11/2022	Sofia Melo Branco

COMMERCIAL IN CONFIDENCE www.vision-box.com

3/32

GATEUYSISCAPII-887_TestPlan_SCP 23/11/2022

Contents

Introduction......................................................................................................................5

Document Overview........................................................................................................................................ 5

Test Plan Details............................................................................................................................................. 5

Summary......................................................................................................................................................... 6

Detailed report................................................................................................................................................. 7

COMMERCIAL IN CONFIDENCE www.vision-box.com

4/32

GATEUYSISCAPII-887_TestPlan_SCP 23/11/2022

Introduction

Document Overview

This Test Plan contains a broad range of scenarios contained within this solution. This document has been generated automatically from the test management platform.

Test Plan Details

Description

Revision

Test Environments

Test Plan containing all the scenarios for a passenger in security checkpoint gate.

None

SQA

COMMERCIAL IN CONFIDENCE www.vision-box.com

5/32

GATEUYSISCAPII-887_TestPlan_SCP 23/11/2022

Summary

Of the 19 Test Scripts contained on GATEUYSISCAPII-887:

Key

GATEUYSISCAPII-888 GATEUYSISCAPII-889 GATEUYSISCAPII-890 GATEUYSISCAPII-891 GATEUYSISCAPII-892 GATEUYSISCAPII-893 GATEUYSISCAPII-894 GATEUYSISCAPII-895 GATEUYSISCAPII-896 GATEUYSISCAPII-897 GATEUYSISCAPII-898 GATEUYSISCAPII-899 GATEUYSISCAPII-900 GATEUYSISCAPII-901 GATEUYSISCAPII-902 GATEUYSISCAPII-903 GATEUYSISCAPII-904 GATEUYSISCAPII-905

GATEUYSISCAPII-906

Summary	#Tests

Standard Flow	2 Boarding Pass Invalid(valid barcode)	2 Flight Validation 1:Departure flight UY+flight date today	2 Flight Validation 2:Departure flight UY+flight date tomorrow	2 Flight Validation 2:Departure flight UY+flight date yesterday	2 Flight Validation 3:Departure flight not UY+flight date today	2 Flight Validation 4:Departure flight not UY+flight date tomorrow	2 Flight Validation 5:Departure flight not UY+flight date yesterday	2 Flight Validation 6:Departure flight UY+flight date NULL	2 Flight Validation 7:Departure flight not UY+flight date NULL	2 Flight Validation 8:Departure NULL+flight date today	2 Flight Validation 9:Departure NULL+flight date tomorrow	2 Flight Validation 10:Departure NULL+flight date Yesterday	2 Flight Validation 11:Departure NULL+flight date NULL	2 Backend call -not response	2 Gate alarm -Tailgate	1 Gate alarm -doors blocked	1 Gate alarm -forced doors	1 Gate alarm -No one detected	1

COMMERCIAL IN CONFIDENCE www.vision-box.com

6/32

GATEUYSISCAPII-887_TestPlan_SCP 23/11/2022

. Detailed report

1.4.1 Test Scenario: GATEUYSISCAPII-888 - Standard Flow

Summary

Passenger scans a Boarding Pass for the current date

Step	Data	Expected Result

1

Passenger	is	instructed present a boarding pass

to	Interior Screen: SC0001

System shall:

-Instructing passenger to scan BCBP -Display Screen SC0001

-The eGate doors are closed

-The eGate semaphore shows a green arrow in the model -The eGate side illumination are green.

2

Passenger presents a boarding pass

Interior Screen: SC0002

System shall:

-Display screen SC0002 informing passenger the BCBP is being read -Detect the boarding pass

-Barcode reader LED: off

3

Passenger waits for barcode validation

System shall:

-Do a request to Service with Boardingpass Data, in order to have the validation of BCBP

-Service receives the request from the system

-Service responds to the system with: code, idTransactionEP and message

COMMERCIAL IN CONFIDENCE www.vision-box.com

7/32

GATEUYSISCAPII-887_TestPlan_SCP 23/11/2022

4	Service responds with "code": 0

Accept Transaction Step

{ "code":

"idTransactionEP": "message": "Boarding

}

Data

0, 0,

correcto"

System shall:

Continue to accept transaction

Expected Result

1

Passenger is instructed to cross doors

Interior Screen: SC0005

System shall:

-Instruct passenger to cross doors -Open doors

-Display screen SC0005

System shall: 2	Passenger cross doors	-Close doors

-Return to initial state

1.4.2 Test Scenario: GATEUYSISCAPII-889 - Boarding Pass Invalid (valid barcode)

Summary

Passenger scans an invalid Boarding Pass

Step	Data	Expected Result

1

Passenger	is	instructed present a boarding pass

to	Interior Screen: SC0001

System shall:

-Instructing passenger to scan BCBP -Display Screen SC0001

-The eGate doors are closed

-The eGate semaphore shows a green arrow in the model -The eGate side illumination are green.

COMMERCIAL IN CONFIDENCE www.vision-box.com

8/32

GATEUYSISCAPII-887_TestPlan_SCP 23/11/2022

2

Passenger presents a boarding pass

Interior Screen: SC0002

System shall:

-Display screen SC0002 informing passenger the BCBP is being read -Detect the boarding pass

-Barcode reader LED: off

3

Passenger waits for barcode validation

System shall:

-Do a request to Service with Boardingpass Data, in order to have the validation of BCBP

-Service receives the request from the system

-Service responds to the system with: code, idTransactionEP and message

4

Service responds with	"code": different from 0

System shall:

Continue to reject transaction

Reject Transaction

Step	Data	Expected Result

1

Passenger is instructed to look for assistance

Interior Screen: SC0004

System shall:

-Passenger should look for assistance -Display screen SC0004

-Return to Initial state

COMMERCIAL IN CONFIDENCE www.vision-box.com

9/32

GATEUYSISCAPII-887_TestPlan_SCP 23/11/2022

1.4.3 Test Scenario: GATEUYSISCAPII-890 - Flight Validation 1: Departure flight UY+flight date today

Summary

Passenger scans a Boarding Pass for the current date

Step	Data	Expected Result

1

Passenger	is	instructed present a boarding pass

to	Interior Screen: SC0001

System shall:

-Instructing passenger to scan BCBP -Display Screen SC0001

-The eGate doors are closed

-The eGate semaphore shows a green arrow in the model -The eGate side illumination are green.

2

Passenger presents a boarding pass

Interior Screen: SC0002

System shall:

-Display screen SC0002 informing passenger the BCBP is being read -Detect the boarding pass

-Barcode reader LED: off

3

Passenger waits for barcode validation

System shall:

-Do a request to Service with Boardingpass Data, in order to have the validation of BCBP

-Service receives the request from the system

-Service responds to the system with: code, idTransactionEP and message

COMMERCIAL IN CONFIDENCE www.vision-box.com

10/32

GATEUYSISCAPII-887_TestPlan_SCP 23/11/2022

{ "code":

4	Service responds with "code": 0	"idTransactionEP": "message": "Boarding

}

0, 0,

correcto"

System shall:

Continue to accept transaction

Accept Transaction

Step	Data	Expected Result

1

Passenger is instructed to cross doors

Interior Screen: SC0005

System shall:

-Instruct passenger to cross doors -Open doors

-Display screen SC0005

System shall: 2	Passenger cross doors	-Close doors

-Return to initial state

COMMERCIAL IN CONFIDENCE www.vision-box.com

11/32

GATEUYSISCAPII-887_TestPlan_SCP 23/11/2022

1.4.4 Test Scenario: GATEUYSISCAPII-891 - Flight Validation 2: Departure flight UY+flight date tomorrow

Summary

Passenger scans a boarding pass for the day after

Step	Data	Expected Result

1

Passenger	is	instructed present a boarding pass

to	Interior Screen: SC0001

System shall:

-Instructing passenger to scan BCBP -Display Screen SC0001

-The eGate doors are closed

-The eGate semaphore shows a green arrow in the model -The eGate side illumination are green.

2

Passenger presents a boarding pass

Interior Screen: SC0002

System shall:

-Display screen SC0002 informing passenger the BCBP is being read -Detect the boarding pass

-Barcode reader LED: off

3

Passenger waits for barcode validation

System shall:

-Do a request to Service with Boardingpass Data, in order to have the validation of BCBP

-Service receives the request from the system

-Service responds to the system with: code, idTransactionEP and message

COMMERCIAL IN CONFIDENCE www.vision-box.com

12/32

GATEUYSISCAPII-887_TestPlan_SCP 23/11/2022

4

Service responds with	"code": different from 0

System shall:

Continue to reject transaction

Reject Transaction

Step	Data	Expected Result

1

Passenger is instructed to look for assistance

Interior Screen: SC0004

System shall:

-Passenger should look for assistance -Display screen SC0004

-Return to Initial state

1.4.5 Test Scenario: GATEUYSISCAPII-892 - Flight Validation 2: Departure flight UY+flight date yesterday

Summary

Passenger scans a boarding pass for the day before

Step	Data	Expected Result

1

Passenger	is	instructed present a boarding pass

to	Interior Screen: SC0001

System shall:

-Instructing passenger to scan BCBP -Display Screen SC0001

-The eGate doors are closed

-The eGate semaphore shows a green arrow in the model -The eGate side illumination are green.

COMMERCIAL IN CONFIDENCE www.vision-box.com

13/32

GATEUYSISCAPII-887_TestPlan_SCP 23/11/2022

2

Passenger presents a boarding pass

Interior Screen: SC0002

System shall:

-Display screen SC0002 informing passenger the BCBP is being read -Detect the boarding pass

-Barcode reader LED: off

3

Passenger waits for barcode validation

System shall:

-Do a request to Service with Boardingpass Data, in order to have the validation of BCBP

-Service receives the request from the system

-Service responds to the system with: code, idTransactionEP and message

4

Service responds with	"code": different from 0

System shall:

Continue to reject transaction

Reject Transaction

Step	Data	Expected Result

1

Passenger is instructed to look for assistance

Interior Screen: SC0004

System shall:

-Passenger should look for assistance -Display screen SC0004

-Return to Initial state

COMMERCIAL IN CONFIDENCE www.vision-box.com

14/32

GATEUYSISCAPII-887_TestPlan_SCP 23/11/2022

1.4.6 Test Scenario: GATEUYSISCAPII-893 - Flight Validation 3: Departure flight not UY+flight date today

Summary

Passenger scans a Boarding Pass NULL departure for the current day

Step	Data	Expected Result

1

Passenger	is	instructed present a boarding pass

to	Interior Screen: SC0001

System shall:

-Instructing passenger to scan BCBP -Display Screen SC0001

-The eGate doors are closed

-The eGate semaphore shows a green arrow in the model -The eGate side illumination are green.

2

Passenger presents a boarding pass

Interior Screen: SC0002

System shall:

-Display screen SC0002 informing passenger the BCBP is being read -Detect the boarding pass

-Barcode reader LED: off

3

Passenger waits for barcode validation

System shall:

-Do a request to Service with Boardingpass Data, in order to have the validation of BCBP

-Service receives the request from the system

-Service responds to the system with: code, idTransactionEP and message

COMMERCIAL IN CONFIDENCE www.vision-box.com

15/32

GATEUYSISCAPII-887_TestPlan_SCP 23/11/2022

4

Service responds with	"code": different from 0

System shall:

Continue to reject transaction

Reject Transaction

Step	Data	Expected Result

1

Passenger is instructed to look for assistance

Interior Screen: SC0004

System shall:

-Passenger should look for assistance -Display screen SC0004

-Return to Initial state

1.4.7 Test Scenario: GATEUYSISCAPII-894 - Flight Validation 4: Departure flight not UY+flight date tomorrow

Summary

Passenger scans a boarding pass for a non UY departure day after departure date

Step	Data	Expected Result

1

Passenger	is	instructed present a boarding pass

to	Interior Screen: SC0001

System shall:

-Instructing passenger to scan BCBP -Display Screen SC0001

-The eGate doors are closed

-The eGate semaphore shows a green arrow in the model -The eGate side illumination are green.

COMMERCIAL IN CONFIDENCE www.vision-box.com

16/32

GATEUYSISCAPII-887_TestPlan_SCP 23/11/2022

2

Passenger presents a boarding pass

Interior Screen: SC0002

System shall:

-Display screen SC0002 informing passenger the BCBP is being read -Detect the boarding pass

-Barcode reader LED: off

3

Passenger waits for barcode validation

System shall:

-Do a request to Service with Boardingpass Data, in order to have the validation of BCBP

-Service receives the request from the system

-Service responds to the system with: code, idTransactionEP and message

4

Service responds with	"code": different from 0

System shall:

Continue to reject transaction

Reject Transaction

Step	Data	Expected Result

1

Passenger is instructed to look for assistance

Interior Screen: SC0004

System shall:

-Passenger should look for assistance -Display screen SC0004

-Return to Initial state

COMMERCIAL IN CONFIDENCE www.vision-box.com

17/32

GATEUYSISCAPII-887_TestPlan_SCP 23/11/2022

1.4.8 Test Scenario: GATEUYSISCAPII-895 - Flight Validation 5: Departure flight not UY+flight date yesterday

Summary

Passenger scans a boarding pass for a non UY departure before departure date

Step	Data	Expected Result

1

Passenger	is	instructed present a boarding pass

to	Interior Screen: SC0001

System shall:

-Instructing passenger to scan BCBP -Display Screen SC0001

-The eGate doors are closed

-The eGate semaphore shows a green arrow in the model -The eGate side illumination are green.

2

Passenger presents a boarding pass

Interior Screen: SC0002

System shall:

-Display screen SC0002 informing passenger the BCBP is being read -Detect the boarding pass

-Barcode reader LED: off

3

Passenger waits for barcode validation

System shall:

-Do a request to Service with Boardingpass Data, in order to have the validation of BCBP

-Service receives the request from the system

-Service responds to the system with: code, idTransactionEP and message

COMMERCIAL IN CONFIDENCE www.vision-box.com

18/32

GATEUYSISCAPII-887_TestPlan_SCP 23/11/2022

4

Service responds with	"code": different from 0

System shall:

Continue to reject transaction

Reject Transaction

Step	Data	Expected Result

1

Passenger is instructed to look for assistance

Interior Screen: SC0004

System shall:

-Passenger should look for assistance -Display screen SC0004

-Return to Initial state

1.4.9 Test Scenario: GATEUYSISCAPII-896 - Flight Validation 6: Departure flight UY+flight date NULL

Summary

Passenger scans a boarding pass without flight date

Step	Data	Expected Result

1

Passenger	is	instructed present a boarding pass

to	Interior Screen: SC0001

System shall:

-Instructing passenger to scan BCBP -Display Screen SC0001

-The eGate doors are closed

-The eGate semaphore shows a green arrow in the model -The eGate side illumination are green.

COMMERCIAL IN CONFIDENCE www.vision-box.com

19/32

GATEUYSISCAPII-887_TestPlan_SCP 23/11/2022

2

Passenger presents a boarding pass

Interior Screen: SC0002

System shall:

-Display screen SC0002 informing passenger the BCBP is being read -Detect the boarding pass

-Barcode reader LED: off

3

Passenger waits for barcode validation

System shall:

-Do a request to Service with Boardingpass Data, in order to have the validation of BCBP

-Service receives the request from the system

-Service responds to the system with: code, idTransactionEP and message

4

Service responds with	"code": different from 0

System shall:

Continue to reject transaction

Reject Transaction

Step	Data	Expected Result

1

Passenger is instructed to look for assistance

Interior Screen: SC0004

System shall:

-Passenger should look for assistance -Display screen SC0004

-Return to Initial state

COMMERCIAL IN CONFIDENCE www.vision-box.com

20/32

GATEUYSISCAPII-887_TestPlan_SCP 23/11/2022

1.4.10	Test Scenario: GATEUYSISCAPII-897 - Flight Validation 7:Departure flight not UY+flight date NULL

Summary

Passenger scans a boarding pass non UY departure and without flight date

Step	Data	Expected Result

1

Passenger	is	instructed present a boarding pass

to	Interior Screen: SC0001

System shall:

-Instructing passenger to scan BCBP -Display Screen SC0001

-The eGate doors are closed

-The eGate semaphore shows a green arrow in the model -The eGate side illumination are green.

2

Passenger presents a boarding pass

Interior Screen: SC0002

System shall:

-Display screen SC0002 informing passenger the BCBP is being read -Detect the boarding pass

-Barcode reader LED: off

3

Passenger waits for barcode validation

System shall:

-Do a request to Service with Boardingpass Data, in order to have the validation of BCBP

-Service receives the request from the system

-Service responds to the system with: code, idTransactionEP and message

COMMERCIAL IN CONFIDENCE www.vision-box.com

21/32

GATEUYSISCAPII-887_TestPlan_SCP 23/11/2022

4

Service responds with	"code": different from 0

System shall:

Continue to reject transaction

Reject Transaction

Step	Data	Expected Result

1

Passenger is instructed to look for assistance

Interior Screen: SC0004

System shall:

-Passenger should look for assistance -Display screen SC0004

-Return to Initial state

1.4.11	Test Scenario: GATEUYSISCAPII-898 - Flight Validation 8:Departure NULL+flight date today

Summary

Passenger scans a Boarding Pass NULL departure for the current day

Step	Data	Expected Result

1

Passenger	is	instructed present a boarding pass

to	Interior Screen: SC0001

System shall:

-Instructing passenger to scan BCBP -Display Screen SC0001

-The eGate doors are closed

-The eGate semaphore shows a green arrow in the model -The eGate side illumination are green.

COMMERCIAL IN CONFIDENCE www.vision-box.com

22/32

GATEUYSISCAPII-887_TestPlan_SCP 23/11/2022

2

Passenger presents a boarding pass

Interior Screen: SC0002

System shall:

-Display screen SC0002 informing passenger the BCBP is being read -Detect the boarding pass

-Barcode reader LED: off

3

Passenger waits for barcode validation

System shall:

-Do a request to Service with Boardingpass Data, in order to have the validation of BCBP

-Service receives the request from the system

-Service responds to the system with: code, idTransactionEP and message

4

Service responds with	"code": different from 0

System shall:

Continue to reject transaction

Reject Transaction

Step	Data	Expected Result

1

Passenger is instructed to look for assistance

Interior Screen: SC0004

System shall:

-Passenger should look for assistance -Display screen SC0004

-Return to Initial state

COMMERCIAL IN CONFIDENCE www.vision-box.com

23/32

GATEUYSISCAPII-887_TestPlan_SCP 23/11/2022

1.4.12	Test Scenario: GATEUYSISCAPII-899 - Flight Validation 9:Departure NULL+flight date tomorrow

Summary

Passenger scans a Boarding Pass NULL departure for the day after

Step	Data	Expected Result

1

Passenger	is	instructed present a boarding pass

to	Interior Screen: SC0001

System shall:

-Instructing passenger to scan BCBP -Display Screen SC0001

-The eGate doors are closed

-The eGate semaphore shows a green arrow in the model -The eGate side illumination are green.

2

Passenger presents a boarding pass

Interior Screen: SC0002

System shall:

-Display screen SC0002 informing passenger the BCBP is being read -Detect the boarding pass

-Barcode reader LED: off

3

Passenger waits for barcode validation

System shall:

-Do a request to Service with Boardingpass Data, in order to have the validation of BCBP

-Service receives the request from the system

-Service responds to the system with: code, idTransactionEP and message

COMMERCIAL IN CONFIDENCE www.vision-box.com

24/32

GATEUYSISCAPII-887_TestPlan_SCP 23/11/2022

4

Service responds with	"code": different from 0

System shall:

Continue to reject transaction

Reject Transaction

Step	Data	Expected Result

1

Passenger is instructed to look for assistance

Interior Screen: SC0004

System shall:

-Passenger should look for assistance -Display screen SC0004

-Return to Initial state

1.4.13	Test Scenario: GATEUYSISCAPII-900 - Flight Validation 10:Departure NULL+flight date Yesterday

Summary

Passenger scans a Boarding Pass NULL departure for the day before

Step	Data	Expected Result

1

Passenger	is	instructed present a boarding pass

to	Interior Screen: SC0001

System shall:

-Instructing passenger to scan BCBP -Display Screen SC0001

-The eGate doors are closed

-The eGate semaphore shows a green arrow in the model -The eGate side illumination are green.

COMMERCIAL IN CONFIDENCE www.vision-box.com

25/32

GATEUYSISCAPII-887_TestPlan_SCP 23/11/2022

2

Passenger presents a boarding pass

Interior Screen: SC0002

System shall:

-Display screen SC0002 informing passenger the BCBP is being read -Detect the boarding pass

-Barcode reader LED: off

3

Passenger waits for barcode validation

System shall:

-Do a request to Service with Boardingpass Data, in order to have the validation of BCBP

-Service receives the request from the system

-Service responds to the system with: code, idTransactionEP and message

4

Service responds with	"code": different from 0

System shall:

Continue to reject transaction

Reject Transaction

Step	Data	Expected Result

1

Passenger is instructed to look for assistance

Interior Screen: SC0004

System shall:

-Passenger should look for assistance -Display screen SC0004

-Return to Initial state

COMMERCIAL IN CONFIDENCE www.vision-box.com

26/32

GATEUYSISCAPII-887_TestPlan_SCP 23/11/2022

1.4.14	Test Scenario: GATEUYSISCAPII-901 - Flight Validation 11:Departure NULL+flight date NULL

Summary

Passenger scans a Boarding Pass NULL departure and flight date NULL

Step	Data	Expected Result

1

Passenger	is	instructed present a boarding pass

to	Interior Screen: SC0001

System shall:

-Instructing passenger to scan BCBP -Display Screen SC0001

-The eGate doors are closed

-The eGate semaphore shows a green arrow in the model -The eGate side illumination are green.

2

Passenger presents a boarding pass

Interior Screen: SC0002

System shall:

-Display screen SC0002 informing passenger the BCBP is being read -Detect the boarding pass

-Barcode reader LED: off

3

Passenger waits for barcode validation

System shall:

-Do a request to Service with Boardingpass Data, in order to have the validation of BCBP

-Service receives the request from the system

-Service responds to the system with: code, idTransactionEP and message

COMMERCIAL IN CONFIDENCE www.vision-box.com

27/32

GATEUYSISCAPII-887_TestPlan_SCP 23/11/2022

4

Service responds with	"code": different from 0

System shall:

Continue to reject transaction

Reject Transaction

Step	Data	Expected Result

1

Passenger is instructed to look for assistance

Interior Screen: SC0004

System shall:

-Passenger should look for assistance -Display screen SC0004

-Return to Initial state

1.4.15	Test Scenario: GATEUYSISCAPII-902 - Backend call - not response

Summary

ResponseCode<>200(Error Calling Service)

Step	Data	Expected Result

1

Passenger	is	instructed present a boarding pass

to	Interior Screen: SC0001

System shall:

-Instructing passenger to scan BCBP -Display Screen SC0001

-The eGate doors are closed

-The eGate semaphore shows a green arrow in the model -The eGate side illumination are green.

COMMERCIAL IN CONFIDENCE www.vision-box.com

28/32

GATEUYSISCAPII-887_TestPlan_SCP 23/11/2022

2

Passenger presents a boarding pass

Interior Screen: SC0002

System shall:

-Display screen SC0002 informing passenger the BCBP is being read -Detect the boarding pass

-Barcode reader LED: off

3

Passenger waits for barcode validation

System shall:

-Do a request to Service with Boardingpass Data, in order to have the validation of BCBP

-Service receives the request from the system

-Service responds to the system with: code, idTransactionEP and message

4

An     error     occurs     calling     the service (ResponseCode<>200)

System shall:

-Notify officer the status FAILURE

-BCBP_CHECK_end with FAILURE in service call -Continue to reject transaction

Reject Transaction

Step	Data	Expected Result

1

Passenger is instructed to look for assistance

Interior Screen: SC0004

System shall:

-Passenger should look for assistance -Display screen SC0004

-Return to Initial state

COMMERCIAL IN CONFIDENCE www.vision-box.com

29/32

GATEUYSISCAPII-887_TestPlan_SCP 23/11/2022

1.4.16	Test Scenario: GATEUYSISCAPII-903 - Gate alarm - Tailgate

Summary

Passenger crosses doors with another passenger

Step	Data	Expected Result

System Shall:

-Display Screen SC0006 to notify passenger 1	Two passengers cross the doors	Interior Screen: SC0006	-The eGate doors are open

-Alarm Notification -Tailgate -Proceed with flow

1.4.17	Test Scenario: GATEUYSISCAPII-904 - Gate alarm - doors blocked

Summary

Accept Transaction Passenger Block Doors

Step	Data	Expected Result

1

Passenger triggers Block Doors Alarm

System Shall:

-Display Screen SC0007 to notify passenger Interior Screen: SC0007	-The eGate doors are open

-Display in Inspector Blocked doors icon alarm and message -Proceed with the flow

COMMERCIAL IN CONFIDENCE www.vision-box.com

30/32

GATEUYSISCAPII-887_TestPlan_SCP 23/11/2022

1.4.18	Test Scenario: GATEUYSISCAPII-905 - Gate alarm - forced doors

Summary

Passenger Forced Doors

Step	Data	Expected Result

1

Passenger triggers Forced Doors alarm

System Shall:

-Display Screen SC0007 to notify passenger Interior Screen: SC0007	-The eGate are closed

-Display in Inspector Forced doors icon alarm and message -Proceed with flow

1.4.19	Test Scenario: GATEUYSISCAPII-906 - Gate alarm - No one detected

Summary

Accept Transaction without a passenger cross the doors

Step	Data	Expected Result

1

Passenger is instructed to cross doors

Interior Screen: SC0005

System shall:

-Instruct passenger to cross doors -Open doors

-Display screen SC0005

2

Passenger does not cross the doors until timeout

System shall: -Close doors

-Return to initial state

COMMERCIAL IN CONFIDENCE www.vision-box.com

31/32

GATEUYSISCAPII-887_TestPlan_SCP 23/11/2022

Appendix A: Approval

The undersigned acknowledge they have reviewed the Test Execution and agree with the approach it presents. Changes to this Test Execution will be coordinated with and approved by the undersigned or their designated representatives.

Signature:	Date:

Print Name:

Title:

Role:

COMMERCIAL IN CONFIDENCE www.vision-box.com

32/32